# Plumbing Assignment 1

Name ______________________

### Description:

For this assignment you will work with three common plumbing materials: galvanized pipe, copper pipe, and plastic pipe.

### Materials:

7" - 1/2" Galvanized pipe

6" - 1/2" Copper Pipe

1 - 1/2 Copper Cap

1 - 1/2" Male Adapter

10"-1/2" PVC Pipe

2 - 1/2" PVC ST 90o Elbow

### Tools:

Steel pipe cutter

Pipe Die

Pipe reamer

Hack Saw

Solder

Propane Torch

Acid and Brush

PVC Cement

### Directions:

Read the directions before beginning the lab. Determine the length of each pipe and write it in the space provided.  Note the completed project should lay flat (all pipes in the same plane).  You may start the assignment at step 1, 2, or 3.

- Determine the correct length of 3/4" galvanized pipe to fit the diagram.  Measurement is from the end of the pipe to the OUTSIDE of the PVC fitting.  Assemble the pipe and fitting.  Cut a _______" Galvanized Pipe nipple. Thread one end with the power threader, ream, and cut to length.  Thread the other end by hand and ream.  Assemble with the provided fitting using pipe sealant.

- Cut a piece of 1/2" copper pipe __________" long to fit the diagram below using a hack saw.  Measurement is from the OUTSIDE of the cap to the OUTSIDE of the PVC fitting.  Length should be adjusted for the copper fittings and the depth the fitting will screw into the PVC Ell.  Assemble the copper pipe and fittings.  Solder the joint.  Screw into the PVC Ell using pipe sealant.

- Cut a piece of 3/4" PVC pipe _________" long to fit the diagram below.  Dimensions are from the OUTSIDE of the fittings.  Glue the joints.

- Using pipe sealant, assemble the union half provided to your galvanized pipe.  Assemble the union and pressure test your project.  CAUTION:  Add air slowly.

- Have your pressure test checked by the instructor and remove the union.  Have your pipe length graded.  Disassemble the project and tape together.

Plumbing Project Grade Sheet

| Criteria | Possible | Score |
| --- | --- | --- |
| Graded During Class: |  |  |
| Galvanized Pipe Length (+/- 1/8”) | 2 |  |
| Copper Pipe Length (+/- 1/8”) | 2 |  |
| PVC Pipe Length (+/- 1/8”) | 2 |  |
| Pressure Test (2 per joint, pass/fail) | 10 |  |
| Graded Later: |  |  |
| Galvanized Nipple |  |  |
| Thread quality (machine thread) | 2 |  |
| Reaming | 2 |  |
| Copper Fitting |  |  |
| Solder joint quality | 2 |  |
| Cut square and deburred | 2 |  |
| PVC |  |  |
| Glue joint quality | 2 |  |
| Assembly/workmanship | 4 |  |
| TOTAL | 30 |  |

9"

(To end of pipe)

Galvanized

Copper

7"

PVC Pipe

10"

(To outside of Ell's)

Not to Scale