# Wood Planter Box

Name: __________________

Date: __________________

## Description:

Students will complete a wood working project by building a wood planter box. The planter box will be big enough to hold 4 – 4” square pots or can be filled with potting soil. This is a great project for Christmas or Mother’s Day.

## Materials:

1” X 6” X 6’ Redwood

5d galvanized box nails

3d galvanized box nails (few)

Wood Glue

Varnish/stain

Sand Paper

Tools:

Table Saw

Power Miter Saw

Drill Press

1” forstner bit

Hammer

Tape Measurer/Ruler/Combination square

## Procedure:

- Obtain a piece of redwood that is 1” X 6” X 6’

- Determine the best use of the lumber.   Where are the knots?   Determine the layout before you begin to cut.

- Crosscut the board at approximately 24” for the bottom.

- Adjust the table saw rip fence so it measures 5” from the fence to the inside of the saw blade

- Rip (cut) the long piece of redwood at 5”.

- From this piece cross cut all the 5” wide pieces (see cutting list – sides, ends).

- Set the table saw to rip at 4 ¼”.  Rip your short board (bottom).  Save the scrap for the feet.

- Crosscut the 4 ¼” board at 17”.

- Crosscut the scrap for the feet.

- All of the pieces have now been cut.  Put your project together without glue and nails.  Does it fit?  If so continue.

- Layout the holes in the bottom.  Drill 4 holes using a Forstner bit evenly spaced under each pot.

- Nail the ends to the bottom using glue and 5d nails.

- Nail the sides to the bottom and ends using glue and nails.  Be sure to properly align the sides (see plan) and insure the ends are square.

- Turn the planter upside down and install the feet as shown the plan.  Use 3 or 4d nails and glue.  Brittle wood may need to be drilled.

- Sand your project down to a smooth finish, DO NOT sand the nails.

- Stain or paint the project for completion

## Cutting List:

| Quantity | Size |  | Material |
| --- | --- | --- | --- |
| 2 | 3/4” X 5 ” X 20” | Sides | Redwood |
| 1 | 3/4” X 4 ¼” X 17” | Bottom | Redwood |
| 2 | 3/4” x 5” X 4 ¼ ” | Ends | Redwood |
| 2 | 3/4”X 5 ¾” X 1” | Feet | Redwood |

## Notes:

## ________________________________________________________________________

## ________________________________________________________________________

## ________________________________________________________________________

## ________________________________________________________________________

## ________________________________________________________________________

## ________________________________________________________________________

## Photo/Drawing:

## Board Layout

A=Side Piece

B=Side Piece

C=End Piece

D=End Piece

E=Bottom Piece

F=Legs

A

B

C

D

E

F

# Wood Planter Box Worksheet

Name: __________________

Date: __________________

- What are the safety precautions when using the table saw?

- When using a table saw why must two people help rip a large piece of wood?

- When using the table saw why must you always measure from the fence to the inside of the saw blade?

- When using the table saw why must the top of the saw blade be 1/8” above the top of the wood?

- What are the safety precautions when using the power miter saw?

- What type of drill bit is used and why?

## Grading Rubric:

| CRITERIA | POSSIBLE | SCORE |
| --- | --- | --- |
| Length | 5 |  |
| Width | 5 |  |
| Holes are properly drilled-no chips, slivers, etc. | 5 |  |
| Holes are lined up straight and evenly spaced | 5 |  |
| Planter box is square and joints fit well. | 5 |  |
| Workmanship (sanding, nails, cut quality, etc) | 5 |  |
| Total | 30 |  |

# Wood Planter Box Teachers Notes:

## Agricultural Standards Met:

6.0	Health and Safety. Students understand health and safety policies, procedures, regulations, and practices, including the use of equipment and handling of hazardous materials:

6.1	Know policies, procedures, and regulations regarding health and safety in the workplace, including employers’ and employees’ responsibilities.

6.2	Understand critical elements of health and safety practices related to storing, cleaning, and maintaining tools, equipment, and supplies.

6.4	Maintain safe and healthful working conditions.

6.5	Use tools and machines safely and appropriately.

6.6	Know how to both prevent and respond to accidents in the agricultural industry.

B1.0	Students understand personal and group safety:

B1.1	Practice the rules for personal and group safety while working in an agricultural mechanics environment.

B1.2	Know the relationship between accepted shop management procedures and a safe working environment.

B1.0	Students understand personal and group safety:

B1.1	Practice the rules for personal and group safety while working in an agricultural mechanics environment.

B1.2	Know the relationship between accepted shop management procedures and a safe working environment.

B2.0 Students understand the principles of basic woodworking:

B2.1 	Know how to identify common wood products, lumber types, and sizes.

B2.2 	Know how to calculate board feet, lumber volume, and square feet.

B2.3	 Know how to identify, select, and implement basic fastening systems.

B2.4	Complete a woodworking project, including interpreting a plan, developing a bill of materials and cutting list, selecting materials, shaping, jointing, and finishing.

## Objectives:

By properly completing this project, students will be able to:

Read a plan to layout dimensions.

Students will identify proper wood type used for the project and discuss options

Students will properly layout project design

Students will safely use a table saw

Students will safely cut wood using the power miter saw

Students will use a drill press to successfully drill the drainage holes

Students will learn the proper way to glue and assemble the project

Students will learn how to sand and stain their planter box

## Alternative Tools/Methods/Materials:

Many simple variations are possible.  Joints may use dado and rabbet cuts if a dado blade is available. Dado and rabbet cuts can also be made with a router or shaper (see alternate plans)

Instead of using nails you could have used #6 x 1 ½ ” deck screws.

Cedar or pine instead of redwood.  Pine would need to be painted.

Discuss other options such as alternative tools to use, splitting the class into groups and work at “stations”, and demonstrating specific skills and safety measures.

## Safety Review:

Table Saw

Power Miter Saw

Drill Press

## Project Time:

| Demonstration: | 1.5 hours |
| --- | --- |
| Build: | 3 hours |

## Demonstration Notes:

- Demonstrate how to select lumber and to layout the project so that you cut around knots and defects in the lumber. Project allows for some waste.   You may wish to simply rip all of the boards.

- Be sure to address safety tips as we work on each machine.

- Demonstrate the “mark-cut-mark-cut” technique used for cross cutting, so pieces are correct length.

- Demonstrate hole layout using a combination or try square.

- Drill holes before legs are glued and nailed on to the bottom piece.

- Make sure that end pieces are placed at the 1-1/2” mark (from the end of the board).

- Make drainage holes evenly spaced and on a straight line.

- Keep scrap pieces separate to avoid confusion.

- Nail sizes.  Explain types and sizes. Why don’t we use 5d on feet?

## Bill of Materials:

Project from: Mike Spiess, Mike Mederos.  Plan developed by Savahanna Rennick.

# Alternate Projects:

There are many variations on this project.  Here are a few….

A version of the project with dado and rabbet cuts.  Good project to introduce more complex wood joints.  Requires a table saw with a dado blade.

This version has a decorative back.  Can be wall mounted.   Use a paper template (another layout skill) to make the back shape.  Cut curves with a ¼” bandsaw.

A simple version made from 1x4.  No table saw needed.  Use 3” pots or fill with soil.

A hanging version made from 1x4.   Use with 3” pots.   Cut ends on bandsaw with ¼” blade. Note use of finish nails (another way to introduce a different type of fastener).

Simple cedar box built around a commercial tray (Home Depot).  Good for indoor use.