Hilmar Ag Department					Name:

Ag Welding

Anvil Project							Date: 				  ID#

Follow the Anvil Project powerpoint to build your anvil, and use this worksheet to record your progress.

- The figure notations (Fig. 1) refer to pictures in the powerpoint.

- For each step, record any information that is requested.

- Have the instructor initial your worksheet after the completion of each step.

| Step | Instructions | Instructor |
| --- | --- | --- |
| # |  | Initials |
| 1. | Cut a 12” section of rail track using the band saw (Fig. 1). Make sure the track is flat on the bed of the band saw (Fig. 2). Your finished piece should look like the picture (Fig. 3). |  |
| 2. | Weigh your section of track and enter the weight here: (Fig. 4) |  |
| 3. | Stamp your ID# on the bottom of your track, in the center, towards one end (This will be the back of the anvil). Make sure it is readable! You may need to grind/sand the spot with a hand grinder to see the numbers better. (Fig. 5) |  |
| 4. | Take a picture of yourself holding your track (Fig. 6) and upload it to your AET as your weekly picture. Be sure to add a caption. (20 Points). |  |
| 5. | Next, cut 2-½” off of the front end of your track (opposite the end with the number stamped on it) with the band saw (Fig 7). Your piece should look like (Fig. 8). Put this piece in your locker RIGHT NOW! |  |
| 6. | From the same end you just cut (front end), measure another 2-1/2” (Fig. 9) and make a cut with the band saw that stops at the base of the track (Fig. 10 & 11). |  |
| 7. | Use the Oxy-Acetylene Torch to make a cut from the band saw cut to the end of the track (Fig. 12), parallel to the base. You should now have a cut piece like the one shown in (Fig. 13). Put this piece in your locker after you have cooled it off. RIGHT NOW! |  |
| 8. | Use the angle grinder with a grinder disc to smooth off the cut you just made (Fig. 14). Then finish it with a sanding disc. Your anvil should now look like (Fig. 15). |  |
| 9. | With the instructor’s help, set up your anvil on the milling machine as shown in (Fig. 16). Make enough passes so that the top of the anvil is smooth (flat) and square. |  |
| 10. | Use layout tools to mark where the holes will be drilled in the base of the anvil. Use the measurements shown in the diagram (Fig. 17) below to mark the holes. See Figures 18-20 in the next slide for examples for marking the location of the holes. |  |
|  | Fig.17 |  |

| 11. | (Fig. 22) shows using the drill press to drill the 4 holes in the base. Make sure the instructor helps you set this up. You will use a ¼” bit to drill pilot holes first, then use a 13/32” drill bit to enlarge the holes. |  |
| --- | --- | --- |
| 12. | Measure 3” from the front end of the anvil (Fig. 23) and make a mark along both edges of the track. Using a piece of bent wire lined up with the 3” mark on the edge to the center of the front of the track (Fig. 24), make a soap stone mark to layout the top of the anvil horn. Flip the wire over to make a mark on the other side (Fig. 25). |  |
| 13. | Part 2 of the Anvil Horn Layout - On the side of the track, hand draw the relief cutout using soapstone as shown in (Fig. 26). The depth of the cutout should be 3” deep. |  |
| 14. | You will use the oxy-acetylene torch to cutout the anvil horn. Start by making the 2 cuts on top of the anvil (Fig. 27). Then make the curved relief cut (Fig 28). Use a chipping hammer to remove the slag from the O/A cuts. Now your anvil should look like (Fig. 29). |  |
| 15. | Use a 5” angle grinder to rough shape the anvil horn (Fig. 30). Check with the instructor as you progress and when given the OK, switch to a 4” grinder with a sanding disc and smooth out the grinder marks. Your anvil should now look like (Fig. 31 & 32). |  |
| 16. | For this step you will need the 2-1/2” piece you cut off the track back in Step 5. Position the piece upside-down on the cut-out section on the rear of the anvil. Use a vice-grip C-clamp and a square to help to position the piece correctly and square with the rest of the anvil (Fig. 33 & 33.5). When you are satisfied that everything is positioned correctly, tack it in place using E6010 rods (Fig. 34 & 35). |  |
| 17. | Weld the pieces together all around (Fig. 36). You will have to make multiple passes to fill in the spaces and gaps (Fig. 37). Be sure to chip the slag and brush the welds between passes. |  |
| 18. | Use the angle grinder to smooth out as much of the welds as you can (Fig. 38 & 39). Some of the weld will be in hard to reach areas for the angle grinder. The pneumatic die grinder or a flapper wheel on a cordless drill may help. |  |
| 19. | Use the band saw to trim 1-1/4” off the front of the base (Fig. 40). |  |
| 20. | Use the angle grinder with a sanding disc (Fig. 41) to smooth out all of your grinder marks. Then use the angle grinder with a flapper wheel (Fig. 42) to polish the bottom of the horn and the top of the anvil. Next you will use the cordless drill with a flapper wheel attachment (Fig. 43) with the correct size wheel to fit the curve under the horn (Fig. 44). Your anvil should look like the anvil in the pictures below (Fig. 45-46). |  |
| 21. | With the instructor’s help, use the sandblaster to thoroughly clean your anvil. It should look like the anvil in the pictures below (Fig. 47-49). |  |

| 22. | Weigh your anvil and enter the weight here: (Fig. 50) |  |
| --- | --- | --- |
| 23. | The next step is to put a finish on the anvil. Start by taping off the top of the anvil (Fig. 51). Then trim the tape as shown in figures 52-53. |  |
| 24. | Set up a place to paint making sure you cover the floor or table from overspray (Fig 54). Lay the anvil on its side and paint the bottom and the side and ends (Fig. 55-56). Be careful to put on a glossy coat but not so much that the paint runs (Fig 56). |  |
| 25. | Once the paint in step 24 has dried to the touch, tip the anvil up on its base (Fig 57) and paint the remaining side (Fig 58). |  |
| 26. | Remove the tape on the top of the anvil. It should now look like Fig 59. Next, sand the anvil tops to remove unwanted paint, and to obtain the desired smoothness. Use the grinder with a flapper wheel or the drill with a flapper wheel (Fig 60-61). |  |
| 27. | Your anvil should look something like the one pictured below (Fig 62). Your anvil project is now complete and you can turn it in to be graded. Answer the questions below. |  |

- What is the difference in weight of your anvil between the initial weight in step 2 and step 22?

(10 pts)

- Which steps in the construction of your anvil contributed to any weight loss of the anvil? (10 pts)

- Which steps would have added weight to the anvil? (10 pts)

- Using the “Railroad-Track-Components-Section” handout, under the “Tee Rail Data Diagram section on page 1, determine the nominal weight per yard of the rail track used to make your anvil. List the dimensions of your track here: (25 pts)

HT -

BW -

HW -

W -

Weight per yard: