Differential Leveling Exercise 1

Agricultural Construction and Soil & Water Management

Skill:  Differential Leveling with a Laser Level

Problem:	You have were hired to survey for a parcel of land in the foothills outside of Bear Valley.  You gathered the information on the field note page as shown below.

| STA | B.S. | H.I. | F.S. | Hub Elev |
| --- | --- | --- | --- | --- |
| TBM-1 | 2.76 |  |  | 100.00 |
| TP-1 | 1.29 |  | 8.78 |  |
| TP-2 | 1.57 |  | 5.39 |  |
| TP-3 | 2.01 |  | 8.53 |  |
| TBM-2 |  |  | 9.41 |  |

Assumptions:	The top of the hub stake represents the soil surface at that location.

The field notes as recorded are precise and accurate.

Procedure:	Complete the field notes to determine the elevation of each of the stations.  When completed, answer the questions below.

Questions:

- If stations TBM-1 and TBM-2 are 500 feet apart, what is the percent slope of the grade to the nearest whole percent?

- What do the following letters/abbreviations stand for and give a definition for each.

TBM:

BM:

BS:

FS:

HI: