AGRICULTURE MECHANICS	NAME:

WOODWORKING

SAFETY RULES	DATE:

SAFETY IN CARPENTRY WORK

Observe the following general safety practices in doing all carpentry work.

1. 	Wear industrial-quality safety glasses. When the work has the possibility for eye injury, be sure to wear safety glasses. Safety glasses should have the Z87.1 log on them to assure they are industrial quality.

2. 	Wear safety-toed shoes. When handling heavy carpentry materials, wear safety-toed shoes to protect your feet and toes.

3. 	When handling carpentry materials, wear a hard hat or a bump cap to protect the head.

4. 	When handling rough carpentry materials, wear gloves.

5. 	When painting, insulating, or working in dusty conditions, always wear an appropriate-type respirator.

6. 	Work clothing should not be loose, baggy, or highly flammable. The big danger is getting loose clothing caught in power machinery. Synthetic clothing is subject to low flashpoints, which may result in severe burns. When working on carpentry jobs, avoid wearing such clothing.

7. 	Use correct hand tools for each job. Hold each tool correctly and use it properly.

8. 	Lift and carry heavy objects correctly. When lifting a heavy object, stand close to the object, bend your legs to pick up the object, and then lift with your legs, not your back.

9. 	Work defensively to avoid falls and falling objects.

10. 	Practice good fire safety where fire hazards exist. Use the appropriate type of extinguisher if a fire should occur: Type A extinguishes-wood and paper fires; Type B extinguishes-gasoline, oils, and paint fires; and Type C extinguishes-electrical fires.

11. 	Remove protruding nails from unused lumber. Prevent puncture injuries by removing nail hazards.

HAND TOOL SAFETY

The following are general safety practices for using hand tools.

1. 	Secure the work. Always use a clamp, a vise, or some means to hold the work. Securing the work prevents it from slipping and frees both hands to hold tools.

2. 	Store tools properly. When tools are not in use, store them safely. Store all sharp-edged cutting tools with the sharp edges down.

3. 	Keep tools clean. Keep all hand woodworking tools clean and free of oil and grease so they will not slip when in use.

4. 	Inspect tools before using. Tools that damaged or have broken handles should be marked unsafe. Do not use them until they have been repaired.

5. 	Use the correct tool for the job. Use each hand woodworking tool only for the job for which it was designed. Forcing a small tool to do the job of a large one may result in injury or tool damage.

6. 	Grip tools firmly. Hold hand woodworking tools--especially the hammer and wrenching bar--securely so that they do not slip and hit someone. Hold tools such as the plane with both hands. Do not wear gloves-they are bulky and make gripping tools difficult.

7. 	Use the correct hammer. Never use a machinist's hammer in place of a carpenter's hammer. Do not strike a hardened steel surface, such as an anvil, with a steel hammer because a small piece of steel may break off and injure someone.

8. 	Avoid hammers with damaged handles. Do not use hammers with broken or split handles. Make sure the handle fits tightly in the hammer head.

9. 	Stand in a safe location. When using a hammer or a hatchet, do not let anyone stand directly in line with the tool's path of movement.

10. 	Hold nails tightly. When starting a nail, hold it tightly so that it will not fly loose and hit someone.

11. 	Remove nails. Before using any cutting tool--including saws, wood chisels, plains, and drills--remove nails or other objects that might destroy the tool's cutting edge.

12. 	Keep cutting tools sharp. Sharp tools insure the best results and the safest operation.

13. 	Cut away from the body. When using tools with sharp edges, cut away from the body.

14. 	Use the correct screw driver. Be sure the screw-driver bit fits properly in the screw slot to prevent it from slipping and causing injury.

15. 	Do not use screw drivers to see if electrical circuits are hot.

16. 	Be careful when gluing. Read labels on containers to check for harmful fumes and highly flammable glues.

PORTABLE POWER TOOL SAFETY

The following are general safety practices for using portable power tools.

1. 	Obtain the instructor's permission. Obtain permission before using any portable power tool.

2. 	Protect your eyes and face. When working with portable power tools, wear industrial-quality safety glasses, or a safety shield.

3. 	Wear proper clothing. Do not wear loose-fitting clothing or jewelry that can get caught in moving parts. Do not roll up long sleeves.

4. 	Protect your hair and scalp. Pull back long hair in a band or a cap to keep it from getting caught in tools.

5. 	Know the tool. Read the operator's manual to learn the tool's applications and limitations as well as its potential hazards.

6. 	Use a respirator. Use a filter-type respirator in dusty conditions.

7. 	Avoid electrical shock. Be sure each tool has a three-blade grounding-type plug or is double insulated. Avoid using power tools in damp or wet places without standing on a dry board. Wear rubber gloves and boots for outdoor jobs.

8. 	Keep the work area clean. Keep the floor free of scraps and oil. Cluttered work areas and benches invite accidents.

9. 	Work only at operating speed. Do not use a power tool before it has reached operating speed or while it is coming to a stop. Never force a tool by applying too much pressure.

10. 	Never stop moving parts abruptly. Once a tool has been turned off, allow it to coast to a stop. Never force the tool into stock to stop it. Be sure the tool has come to a complete stop before laying it down.

11. 	Unplug tools properly. Unplug each tool from the power source after use and when you service or change accessories in a safe place when not in use.

12. 	Store unused accessories. When not in use, store tools and accessories in a safe place.

13. 	Use both hands to hold and guide saws and drills.

14. 	Stand in a safe location. To avoid being hit if the tool kicks back, do not stand directly behind the equipment.

SAFETY IN POWER WOODWORKING

The following are general safety practices for using all stationary power tools.

1. 	Obtain instructor's permission before you use any stationary power tool.

2. 	Wear proper clothing. Do not wear loose-fitting clothing or jewelry that can get caught in moving parts. Do not wear gloves. Do not roll up long sleeves.

3. 	Protect your scalp and hair. Pull back long hair in a band or a cap to keep it from getting caught in moving machine parts.

4. 	Keep guards in place. Guards cannot protect you if they are not in place and in proper working order.

5. 	Avoid shock. Be sure that equipment is properly grounded. Do not use power tools in a damp or wet place without standing on a rubber mat.

6. 	Keep the work area clean. Keep the floor free of sawdust, grease, oil, and scraps. Cluttered work areas and benches invite accidents.

7. 	Unplug the machine. The electrical service to each machine must be disconnected when it is not in use, while it is being serviced, and when the accessories are being changed.

8. 	Work only at operating speed. Do not use a power tool before it has reached operating speed or while it is coasting to a stop.

TABLE SAWS

Safety Practices for Using Table Saws

Before studying the specific safety practices for using table saws, review the general safety practices for using stationary power tools.

1. 	Do not roll up long sleeves. Do not wear loose fitting clothing such as coats and ties.

2. 	When operating the table saw, do not wear gloves. Gloves are bulky and may easily be caught on parts of the table saw, thus creating an additional safety hazard.

3. 	Remove rings, bracelets, and other loose-fitting jewelry which have the potential of getting caught on the saw or materials.

4. 	Check the saw blade periodically for broken teeth and cracks.

5. 	When operating the table saw, stand to one side of the saw blade.

6. 	Make certain the blade guard, splitter, and anti-kickback device are used for all possible sawing operations.

7. 	Never reach over the saw blade to remove or hold down a piece of stock.

8. 	Avoid awkward work positions that may result in slips and contact with the saw blade.

9. 	Never raise the saw blade more than 1/4" above the material being cut. Lower the blade below the table top before you leave the work area.

10.	Do not attempt to rip round or irregularly shaped stock on the table saw.