Name: __________________

Period: __________________

# Pre-Operation Check

All equipment must be checked at the beginning of each day prior to operation.  The operator is responsible for checking the equipment for safety issues, maintenance needs and overall health of the equipment.  The pre-op check should be used at the beginning of each shift and allows for students to learn about the features on tractors and implements.  This activity is a foundation to learning troubleshooting skills in machinery maintenance and management.

# Equipment:

Implement (if being used)

Tractor

# Procedure:

- Use pre-operation checklist to examine equipment

- Identify issues or needs on the equipment

- Check safety components

- Make sure equipment is in proper operating condition

- Setup implement properly before taking equipment out to the field

- Turn in pre-operation check sheet to the instructor

- Notify instructor for any safety or maintenance issues on the equipment

# Teaching Notes

- Students should work independently to answer the questions on the checklist

- The teacher should demonstrate the proper way to check the equipment prior to students completing the checklist

- Correct any issues on the equipment prior to operating the tractor/equipment

- This activity should be used each time students operate a tractor

- Use the below pre-op checklist for this activity

Agricultural Equipment Operational and Safety Checks

Tractor/Equipment # _______________ Make/Model________________________

Hours_____________ Date_______________ Inspected by___________________

Check                                             				Note Specific Problem

- Engine oil

- Hydraulic/power train fluid

- Coolant level

- Fuel level

- Air cleaner

- Clean grill & radiator screen

- Safety shields in place

- Left & right brakes same travel

- Tire inflation and condition (2 cleats touching at outside for bias tires, 3 cleats touching at outside for radial)

- Seat belt in workable condition

- Operator seat function

- ROPS

- SMV sign attached

- No leaks, damaged or missing parts

- Safety start system operational

- Indicators, gauges & meters working

- Turn, hazard & operation lights operational

- Danger, warning and caution labels in place and readable

- Clean operators station

- Correct 3 point hitch pins

- Correct drawbar position, hitch pin & lock

- Proper PTO setup

- Safety chain for trailing equipment if operating on public road

# Operational Checks

- Oil pressure

- Engine temperature

- Brakes

- Hydraulic response (2 to 3 seconds to lift from bottom to top at mid throttle)

- Steering

- Power train, clutch, shifting and mechanical front wheel drive

# Post Operation Checks

- Clean tractor

- Safety concerns

- Operational problems

- Leaks, noises, missing parts

- Maintenance required