Name: __________________

Period: __________________

# 3 Point Hitching Activity

All equipment must be checked at the beginning of each day prior to operation.  The operator is responsible for checking the equipment for safety issues, maintenance needs and overall health of the equipment.  Maintain a safe environment, supervise students throughout the activity.

# Equipment:

Implement

Tractor

# Procedure:

- Setup a driving course and demonstrate to students the objectives of this activity

- Students will need to do a pre-operation check prior to operating the equipment

- Practice three-point hitching.  Hook up equipment.

- Use the proper sequence below when hitching:

Lower left, lower right, top link

- Make sure to adjust implement properly and use the implement stabilizers.

- Practice maneuvering equipment around the field.

- Practice backing in a straight line.

- Lower the implement to the ground and use the proper sequence when unhitching equipment.

Top link, lower right, lower left

- Safely park your tractor in the specified stall.

- List the Make and Model of the tractor you operated:

Make __________ Model ___________

# Teaching Notes

- Students should practice hitching equipment multiple times

- This activity can be used with multiple pieces of equipment

- Students should be able to identify what category hitch is on the equipment

- Large pieces of equipment are incredibly dangerous to hitch