Name: __________________

Period: __________________

## Hitching

Purpose: Continue to gain driving proficiency, learn safe hitching practices and understand how common tractor hitching systems work.

- You will need to operate each different tractor provided for this lab.

- Do a pre-start check on TWO tractors.  Note any problems below. IF A TRACTOR IS UNSAFE OR NOT IN DRIVING CONDITION -- DO NOT DRIVE!  Record the number of each tractor you checked below.

| Make/Model | Condition |
| --- | --- |

- Review and identify the components of the following tractor systems as you do the daily check: Cooling, Fuel, Intake, Exhaust, Charging, and Starting.

- What type of hitch is found on each piece of equipment?

| Tractor | Implement | Hitch (type/size) |
| --- | --- | --- |

- Hitch each tractor to the provided implement.

- Raise and move the implement to the designated location and disconnect it.  Repeat as needed for practice.

- Remember Listen, Look, and Live. If time and equipment allow, practice precision driving.

- What are the safety considerations for connecting implements?

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

### Key Points:

At the conclusion of the lab activity you should be able to:

- Preform a daily service on a tractor.

- Understand and operate 3 point and hydraulic controls.

- Correctly connect implements to a 3 point or drawbar.

- Describe the safety issues associated with hitching and operation of a tractor with attached implements.

- Identify common tractor controls and engine systems such as injection pump, radiator, intake manifold, exhaust manifold, alternator, battery, starter motor.

Notes:

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________

_____________________________________________________________________________________