Name: __________________

Period: __________________

# Hydraulic Activity

Tools Required: Pressure tester, hydraulic analyzer, calipers, steel tape, tractor with remote hydraulics.

# Exercise #1

Tractor Used:

Identify the type of hydraulic system:

- Check hydraulic pressure using a pressure gauge at remote coupler by holding the auxiliary control valve handle in the extend position until the relief valve functions. (Run the tractor at 1000 RPM).            PSI________

- Hook an implement with auxiliary hydraulics to the tractor and check the pressure required to lift the implement.  Read the pressure gauge as the implement is lifting.  (Run the tractor at 1000 RPM).    PSI_________

- Record the time it takes to lift the implement in seconds.  Seconds __________

- Measure the diameter of the exterior of the hydraulic cylinder and estimate the piston diameter.  Usually cylinders are in ½ inch increments.

Piston diameter_________

- Measure the stroke length of the hydraulic cylinder.  _________inches

- Calculate to force needed to lift the implement.  Force __________

- Why is there a difference in the PSI between #1 and #2?

- What type of hydraulic system does this tractor have?

Exercise #2

Tractor Used:

Identify the type of hydraulic system:

- Check and record the hydraulic flow (GPM) using the flow tester at the following PSI and Speeds:

Flow at 2000 psi:  						Flow at 1500 engine RPM

| Pressure (psi) | Flow (GPM) |
| --- | --- |
| 500 |  |
| 1000 |  |
| 1500 |  |
| 2000 |  |

| Tractor RPM | Flow (GPM) |
| --- | --- |
| 1000 |  |
| 1500 |  |
| 2000 |  |
| Max: |  |

- How does flow change with RPM?

- What horse power is required to develop full GPM and PSI?

- What type of hydraulic system does this tractor have?

Horsepower = PSI x GPM

1714

Exercise # 3

Tractor used:____ __________________________________________________

Identify the type of hydraulic system:_____________________________________________

- Connect the tractor to a hydraulic cylinder and record the time to extend and retract the hydraulic cylinder with the tractor running at 1500 RPM.  Extend seconds ________          Retract seconds ___________

- Measure the hydraulic cylinder diameter and estimate the piston diameter.  Diameter ___________

- Measure the diameter of the rod and calculate the net square inches surface for the rod side of the piston.  Square inches _______

- Measure the stroke of the hydraulic cylinder.  Stroke________

- Calculate the extension and retraction speed of the cylinder if the pump is delivering ___ gpm.  Seconds extension_____ Seconds retraction______

- What is the difference from the actual to calculated cylinder extension and retraction speeds?

- Test maximum tractor hydraulic pressure.  ___________psi

Calculations:

Force developed by a hydraulic cylinder

Piston Sq Ins = 0.7854 X diameter squared

Force = Area, square inches piston X PSI

Travel speed of a hydraulic cylinder piston

Travel speed, inches/minute = Volume of oil to cylinder, cubic inches/ min

Piston area, square inches

Inches/second = Inches/minute X 1 min

60 seconds

231 cubic inches/gallon