Name: __________________

Period: __________________

# Tractor Safety Test

TRUE/FALSE: In the blank before each question write “T” if the statement is true or “F” if the statement is false.

| ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____ ____________ ________ ____ ____ ____ ____ ____ | 1. 2. 3. 4. 5. 6. 7. 8. 9. 10. 11. 12. 13. 14. 15. 16. 17. 18. 19. 20. 21. 22. 23. 24. 25. 26. 27. 28. 29. 30. 31. 32. 33. 34. 35. 36. 37. 38. 39. 40. 41. 42. 43. 44. 45. 46. 47. 48. 49. 50. | T F Seat belt use is required if the tractor is equipped with ROPS. T F One should always start the engine from the operator seat. T F One should always face the machine when leaving the machine. T F Preoperational checks should be performed daily or whenever a new operator takes over on a tractor. T F Carrying a passenger on the tractor is not OK unless there is an additional seat. T F All tractors have seats that are adjustable for operator comfort. T F Loader buckets should always be carried 5 to 6 feet above the ground level, so you have a clear view below the bucket. T F You should always back a tractor up a steep slope. T F When pulling with a chain, you should connect the chain to the tractor three-point lift and use the hydraulics to help lift to gain leverage. T F PTO guards and shields must always be in place before using the PTO. T F You should always shutdown the tractor and lower or block up an implement before working on the implement. T F When the tractor and implement is parked the hydraulics must be relieved of all pressure. T F ROPS are required unless you operate on in orchards, in barns that are too low for clearance or vineyard with overhead trellis. T F Brakes should be unlocked for highway travel. T F When turning off a paved road you should first slow down and then pull off the road. T F Mechanical front wheel drive on a tractor must be disengaged when traveling on paved or gravely road surfaces. T F You should mount and dismount the tractor with a 3 point stance facing the operator platform. T F You should wear snug fitting clothing when operating equipment. T F When operating on a public road you should try to keep one wheel on the shoulder and one wheel off the road to allow traffic to pass. T F It is ok to motion vehicles to pass when you are operating equipment on public roads. T F You should inspect the field for hazards before working it with a tractor. T F Implement drawbar hitch pins need to be secured only for road travel. T F PTO shaft shields should turn freely over the PTO shaft. T F Tractors and specifically the operator platform should be kept clean to avoid possible interference of obstacles with operator controls. T F Correct tire pressure depends on the weight of the tractor, tire type, ballast and implements. T F Bias ply tractor tires that are properly inflated would have the outside edge of two cleats hitting the ground while properly inflated radial tires would have the outside edge of three cleats hitting ground. T F When 3 point hitching you should have a helper hold the lower links to help connect the implement. T F Tractors are more stable when the three point implement (like a mower or disc) is carried as high as possible during transportation. T F When towing equipment on a public road an appropriate safety chain is required. T F Once you have lowered an implement and are working in the field you don’t need to look at it again until you raise it because you will hear if anything breaks. T F When driving on public roads you are subject to the same rules of the road as a motor vehicle. T F Before operating equipment, you should walk completely around the tractor and inspect for missing or loose bolts, damaged parts and other obstacles. T F Tractors have a high center of gravity. T F You should reduce speed, (throttle down), while turning to avoid a rollover. T F When disconnecting the hydraulic lines of a towed implement, you should lower the implement to the ground and leave the wheels just touching the ground so that the hydraulic lines are not pressurized. T F Increasing front ballast will stabilize the tractor when the three point implement is raised. T F When operating on a public road you must have a SMV sign in place and use the emergency flashers. T F When hitching and unhitching one should use a level spot to avoid possible movement of the tractor and equipment. T F When slowing down a tractor you should first lower the engine speed (RPM) then apply the brakes. T F Steering brakes should be checked to see that they both have the same adjustment and then left locked together for road travel. T F You should always operate the tractor controls from the operator’s seat. T F Starting fluid helps the starting of a cold diesel engine in all situations. T F Jump starting a tractor is safe as long you hook up cables correctly to each battery. T F An instrument panel check will tell you if hydraulics work properly. T F One should never tow a trailer or implement down the road that weighs more than 2X the tractor. T F A PTO shield will protect you from any hazard that might occur if your are near the shaft. T F Roll over protection may be put down when operating around livestock building that are lower than the ROP’s. T F Field surfaces should be checked before operating in a field to make sure that surfaces and trash are not an obstacle in operation. T F The accepted distance for operating a tractor near ditch is equal to the depth of the ditch. T F Acceptable tractor speed around livestock and storage facilities would be 5 to10 MPH. |
| --- | --- | --- |