## Ballasting Tractors

Purpose:  To understand the procedure to properly ballast a tractor and to understand the effects of ballast on tractor operation and performance.

Equipment Needed:

Truck scale

Tape measure

Tire pressure gauge

3 point implement

Tractor

Operator’s Manual

Fork lift

Chain

Note: Portions of this lab are adapted from John Deere Operator’s Manuals.

### Background:

100
lbs

? lbs

100 in.

50 in.

Consider the “teeter-totter” illustrated above.  For the beam to be balanced, the unknown weight must be __________ lbs.  The ratio of the weights is simply the ratio of the distances.   Assuming the beam weighs 10 lbs what is the weight on the balance point?  __________ lbs.

## Ballasting for Power

Proper ballast will control wheel slip.

### Attaining Optimum Performance

Before adding ballast to your tractor, consider these important factors to attain optimum performance:

- Total tractor weight and static weight split (percent of static weight on front and rear axles)

- Type of ballast used (cast weight or liquid)

- Tire inflation pressures

### Recommended Weight Split

Tractor should be weighed to accurately determine amount and type of ballast. Weight should be distributed depending on how tractor is equipped and the conditions in which it will be operated.

### Correct Ballast

Use no more ballast than necessary, and adjust ballast as tractor use changes.

For correct ballast, measure amount of travel reduction (% slip) of the drive wheels. Under normal field conditions, travel reduction should be 10-15 percent (8-12 for MFWD tractors). Add more weight to drive wheels if slip is excessive. If there is less than minimum percent slip, ballast should be removed, unless needed for stability.

Correct ballast allows for most efficient use of tractor's available power and will not make up for an implement which is too big for the tractor. Adding ballast will not improve performance, if engine speed falls below rated speed and/or wheel slip is not beyond the recommended range.

### Correct Inflation Pressure

Inflate tires to correct pressure to carry load on each axle for optimum tractive performance.

NOTE: See Wheels, Tires, and Treads Section for inflation pressure charts.

### Selecting Ballast Carefully

NOTE: Radial-ply and bias-ply tires use same ballasting procedures.

### Factors Determining Amount of Ballast

- Soil surface-loose or firm

- Type of implement-integral/semi-integral or towed

- Travel speed-slow or fast

- Tractor power output-partial or full load

- Tires-single or dual; small or large

Pulling a lighter load at a higher speed is more economical and more efficient than pulling heavier loads at a lower speed.

| Too Little Ballast | Too Much Ballast |
| --- | --- |
| Excessive wheel spin | Soil compaction |
| Power loss | Power loss |
| Tire wear | Increased load |
| Fuel waste | Fuel waste |
| Lower productivity | Lower productivity |

### Ballast Limitations

IMPORTANT: Tractor weight exceeding heavy ballast limits should be avoided and may void the warranty due to "overload" conditions.

Ballast should be limited by the lowest of either tire capacity or tractor capacity. Carrying capacity of each tire should not be exceeded. If a greater amount of weight is needed, a larger single tire or duals should be considered.

### Calculating Ballast for Optimum Performance

Tractor weight and ballasting requirements vary depending on tractor equipment, tire inflation, and operating conditions.

Follow these instructions to determine ballasting requirements

| Step 1 | Determine recommended total working weight and weight split for application ballast. |
| --- | --- |
|  | Total tractor weight requirement is found by multiplying the tractor PTO horsepower times the appropriate kilogram (pound) per horsepower. Kilograms (pounds) per horsepower requirements vary according to soil and load operating conditions. |

| Soil Type: | kg (lb)/PTO-hp |
| --- | --- |
| Light | 54 kg (120 lb) |
| Medium | 59 kg (130 lb) |
| Heavy | 66 kg (145 lb) |

|  | The total tractor weight necessary to maintain the recommended wheel slip range is influenced by soil type and travel speed. The ballast levels are based upon the following speeds: km/h (mph). |
| --- | --- |

| Soil Type | Travel Speed |
| --- | --- |
| Light | 8.7 km/h (5.4 mph |
| Medium | 7.7 km/h (4.8 mph) |
| Heavy | 7.2 km/h (4.5 mph) |

|  | If different travel speeds are to be utilized, more or less weight will be needed. Higher speeds will not require as much weight. |
| --- | --- |
|  | In most cases, 59 kg (130 lb)/PTO horsepower will be enough weight to pull heavy loads. Example: |
|  | PTO hp 170 x 59 kg (130 lb) = 10,024 kg (22,100 lb) |
|  | 10,024 kg (22,100 lb) is the total tractor working weight that would be needed at 59 kg (130 lb)/PTO horsepower. |
|  | Next, determine how much of the 10,024 kg (22,100 lb) weight needs to go on the front and how much on the rear of the tractor. Factory recommended weight splits are: |

| Front Axle | Towed | Semi-Integral | Integral |
| --- | --- | --- | --- |
| 2WD (Front/Rear) | 25/75 | 30/70 | 35/65 |
| MFWD (Front/Rear) | 35/65 | 35/65 | 35/65 * |

|  |  | * Front weight requirements are determined by the weight of the hitch-mounted implement. Enough front weight needs to be added to maintain steering control. |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | In this example, the MFWD tractor will be pulling a towed implement. The recommended 35% of the total weight should be on the front and 65% should be on the rear of the tractor. |  |  |  |  |
|  |  | Total Front Weight: | 10,024 kg (22,100 lb) x 0.35 = 3509 kg (7735 lb) |  |  |
|  |  | Total Rear Weight: | 10,024 kg (22,100 lb) x 0.65 = 6516 kg (14,365 lb) |  |  |
|  | To determine how much weight needs to be added to the front and rear, go to Steps 2 and 3. |  |  |  |  |
|  | NOTE: Additional ballast may be required due to loss of tractor stability. |  |  |  |  |
| Step 2 | Determine current tractor weight by using the Operator's Manual or by using a scale. Find the chart with the correct tractor model and tire size. |  |  |  |  |
|  | The tractor in the example has MFWD and is equipped with 480/80R46 dual tires and has no front weights. The weight guide lists: |  |  |  |  |

| Total Front Weight | Total Rear Weight |
| --- | --- |
| 2942 kg (6485 lb) | 5202 kg (11,468 lb) |

| Step 3 | Subtract current tractor weight (determined in Step 2) from the recommended total weight and weight split (determined in Step 1) and add ballast as required. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Front Ballast: | 3509 kg (7735 lb) - 2942 kg (6485 lb) = 567 kg (1250 lb) |  |
|  |  |  | Rear Ballast: | 6516 kg (14,365 lb) - 5202 kg (11,468 lb) = 1314 kg (2897 lb) |  |
| IMPORTANT: |  |  | There is a weight transfer effect when adding front weights since they are mounted forward of the front axle. |  |  |
| Step 4 |  | Adjust tire inflation pressures for optimum performance. (See TIRE INFLATION PRESSURE GUIDELINES in the Wheels, Tires and Treads section.) To get optimum performance from the tractor, inflation pressures must be correctly adjusted to gain maximum traction and minimum soil compaction. Ground pressure "soil compaction" is equal to 6.89-13.79 kPa (0.069-0.138 bar) (1-2 psi) higher than tire inflation pressure. |  |  |  |
| Step 5 |  | The final determining factor of correct ballast is a wheel slip measurement in the field. Under normal conditions wheel slip should be: |  |  |  |

| Tractor | Percent |
| --- | --- |
| 2WD | 10-15 |
| MFWD | 8-12 |

|  | Radar is needed to calculate average wheel slip. When tractor is equipped with radar, wheel slip will be displayed in the vehicle monitor window. Checking wheel slip manually is possible but will only show slip in one area of the field. (See MEASURING WHEEL SLIP-MANUALLY.) Field conditions vary from one area to another. Maintain the correct average wheel slip to maintain optimum traction performance. |
| --- | --- |
| Step 6 | Add more weight to drive wheels if slip is excessive. If there is less then minimum slip, weight should be removed. NOTE: To extend drivetrain life, never add ballast that results in continuous full power loads below 6.6 km/h (4.1 mph). |

### Part I – Basic Tractor Data Collection:

- Tractor Make: 	____________  Model: ___________

- PTO HP:	____________

- 3 point implement: 	___________________________

- Rear Tire Size: 	___________________________

- Front Tire Size: 	___________________________

- Maximum load for these tires (see manual, tire, or chart in appendix) ______ lbs.

- Actual Rear Tire Pressure 	Left __________  Right _________

- Actual Front Tire Pressure 	Left __________  Right _________

- Existing ballast (check if present):

___ Rear wheel weights

___ Front wheel weights

___ Front weights.  Number: ___________

___ Front Liquid Ballast

___ Rear Liquid Ballast

___ Other (list) _______________________________

### Part II – Implement Data

| - Distance from center rear axle to 3 point implement connection on draft links: | ______ in. |
| --- | --- |
| - Implement center of gravity in inches from the 3 point draft link connection. Estimate the center of gravity of the implement using a chain and forklift (or hoist) and mark with chalk. | ______ in. |
| - Total distance from the implement center of gravity to center of rear axle. | ______ in. |

- Enter the measurements in the diagram below.  Measure from the center of the rear axel to: 1) front axel, center of front weights, center of tanks (if equipped), and center of the implement.  Record your measurements below.

### Part III – Ballasting for Stability (Safety)

To maintain stability when driving with the implement raised the front weight of the tractor with the implement raised should be at least 50% of the weight with the implement lowered.

### Part IV – Weighing the Tractor

To accurately set the ballast on a tractor you must weigh the tractor.

| Using the scale determine the following: |  |
| --- | --- |
| Front Axle Weight |  |
| Total Tractor Weight |  |
| Rear Axle Weight (by subtraction) |  |
| Connect the 3 point implement to the tractor and weight the tractor: |  |
| Front axle weight with implement raised |  |
| Total tractor weight with implement |  |
| Rear axle weight with implement raised (subtract) |  |
| Implement weight (subtract tractor weight) |  |
| Calculate the weight on each rear tire. |  |

### Part V – Ballasting for Power

Review the references in the appendix.

### Worksheet to Calculate Ballast Changes

IMPORTANT: Ballast should not exceed weight required to result in recommended percent slip at 6.6 km/h (4.1 mph) MINIMUM.

NOTE: Complete this entire worksheet before adding or changing any ballast, or air pressures.

|  | Front | Rear | Total |
| --- | --- | --- | --- |
| 1. Determine desired weight split for your operation. | _____ % | _____ % |  |
| 2. Record desired weight of tractor (See Ballasting Guide, based on PTO Hp). |  |  | ________ |
| 3. Percent of Weight Split from Step No. 1 multiplied by Step No.2 (Desired Weight) results in Total Front Weight. For Rear Weight, subtract front weight from Total Weight. | ________ | ________ | ________ |
| 4. Weight of tractor as determined from the Unballasted Tractor Weight Chart or weight from scale. | ________ | ________ | ________ |
| 5. Ballast needed (subtract tractor weight in Step No. 4 from desired weight in Step No. 3 | ________ | ________ |  |
| 6. Add ballast. | ________ | ________ |  |
| 7. Add ballast from Step No. 6 to weights from Step No. 4. (Final Weight) | ________ | ________ |  |
| 8. Set tire pressure for operating conditions using weights from Step No. 7. (See appropriate inflation pressure table in manual or reference). | ________ | ________ |  |
| NOTE: You are now ready to test for wheel slippage. See Measuring Wheel Slip (Appendix). |  |  |  |

### Key Points:

At the conclusion of the lab activity you should be able to:

- Understand the forces involved in ballasting.

- Describe the difference between ballasting for safety and ballasting for power.

- Determine tire size and type by reading the data on the tire.

- Determine the maximum load of a tire.

- Determine proper tire inflation levels.

- Preform the calculations to determine the proper weight of a tractor.

Adapted from John Deere Tractor Manuals