# Grain Drill Calibration

Name:_____________________

Date: _____________________

## Description

Grain drill require calibration to ensure that the seeding rate is correct.  Overseeding wastes seed and under seeding will reduce yield.

Make/Model of the grain drill: ________________________________

## Tools/Materials

Drill Manual

Scale

Floor Jack and Jack Stands

Chalk

Tarp (or baggies and rubber bands)

Shopping bag

Fiberglass Tape

Grain

## Generalized Procedure:

Using drill manual set the drill to the desired rate in pounds/acre for the type of seed (ex. wheat).

Fill the drill hoppers with several inches of seed.

Calculate the width of the drill by multiplying the number of openers x the opener spacing.

Capture the seed for a fixed distance.  Use a minimum of 100 feet, 200 feet is better.

Calculate the area covered in acres.

Weigh the captured seed (lbs.) and divide by the area (in acres).

Adjust the drill (if needed) and repeat the check.

## Formula:

Acres = width (ft) x distance (feet) ÷ 43560 sq ft / acre

Pounds/Acre = pound of grain captured ÷ acres covered

Tire Circumference (feet) = Circumference (inches) ÷ 12

## End Wheel Drill Procedure:

- Jack up the end wheels and place jack stands under the drill so the wheels can be turned by hand.   DO NOT leave the drill resting on the jack.

- Using the manual set the drill to the desired setting.

- Use the tape to measure the circumference of the end wheel and calculate the distance the drill will travel in one revolution of the wheel in feet.

- Calculate the width of the drill (openers x spacing) in feet.

- Place a tarp under the drill to catch the seed.

- Fill the hoppers with enough seed to cover the metering devices with several inches of seed.

- Turn the end wheel 100 times.  Note most drills are divided with half of the drill be driven by one wheel.   You may choose to calibrate ½ of the drill at a time or rotate both wheels.  CAUTION:  Some drills will have a different number of openers on each side so the “width” will be different.

- Pour the seed from the tarp into the shopping bag and weigh the seed.

- Complete the calculations below:

| - Width (feet) |  |
| --- | --- |
| - Tire Circumference (feet) |  |
| - Revolutions |  |
| - Acres covered (calculate) |  |
| - Grain captured (pounds) |  |
| - Pounds Per Acre |  |

- Adjust the drill as needed.

## Drill Procedure (driving):

Some drills are difficult to operate in the shop.   This method can be used with all drills and involves capturing the seed as the drill is driven in the field.

- Using the manual set the drill to the desired setting.

- Calculate the width of the drill (openers x spacing) in feet.

- Remove the seed tube from each opener.

- Place a baggie over each seed tube and secure with a rubber band.

- Fill the hoppers with enough seed to cover the metering devices with several inches of seed.

- Layout a 200’ course and drive the drill.   Seed should be captured in the baggies.

- Empty the baggies into shopping bag.  NOTE:  You can weigh the baggies first noting the location of each drill to see if the drills are uniform.

- Complete the calculations below:

| - Width (feet) |  |
| --- | --- |
| - Tire Circumference (feet) |  |
| - Revolutions |  |
| - Acres covered (calculate) |  |
| - Grain captured (pounds) |  |
| - Pounds Per Acre |  |

- Adjust the drill as needed.

## In Field Checking

- Operators can check the seeding rate in the field using this method.

- Fill the hoppers full.

Drive several lengths of the field.

Refill the hoppers and record the number of sacks used.

Calculate the area planted (drill width x field length x passes ÷ 43560).

Divide the weight (bags x bag weight) of the replaced grain by the area in acres.

# Grain Drill Calibration Teachers Notes:

## Objectives:

By properly completing this activity, students will be able to:

Calculate seeded area

Understand the concept of a seeding rate

Understand the relationship of seed dropped to plant population

Demonstrate how to check the calibration of the grain drill

## Teaching Methods

End wheel drills can be checked in ½ as each wheel drive ½ the drill so two teams of students can calibrate at a time.    Review the math.  Remember to point out that the acres covered will be a fraction of an acre (typically a 10th or so).

## Resources:

https://youtu.be/TLv6SmqlYlU

## Safety Review:

How to properly use jack stands