## Measuring Wheel Slip

1. Mark a rear tire.

2. Mark a starting point on the ground with the tractor moving at normal operating speed and implement lowered in the ground.

3. Follow tractor and mark the ground again where marked tire completes 10 full revolutions.

4. Repeat procedure with implement raised at the same working speed. Count revolutions between same two marks.

5. Use second count and chart to determine slippage.

6. Adjust ballast or load to give correct slippage.

NOTE: Ideal slippage is 10-15 percent (8-12 percent for tractors with MFWD).

NOTE: Available horsepower is greatly reduced when wheel slip drops below minimum percent.

| Wheel Slippage Chart |  |  |
| --- | --- | --- |
| Wheel Revolutions | % | Result |
| (Step 4) | Slip |  |
| 10 | 0 | Remove Ballast |
| 9-1/2 | 5 |  |
| 9 | 10 | Correct Ballast |
| 8-1/2 | 15 |  |
| 8 | 20 | Add Ballast |
| 7-1/2 | 25 |  |
| 7 | 30 |  |