# Setting the Proper Tractor Tire Pressure

Name: __________________________

Section/Period: ___________________

Understanding of the proper tractor inflation is important to optimizing tractor performance.

| Over inflation may cause: - Loss of traction - Excessive soil compaction - Fast wear both in field and on road - Higher fuel consumption - Fewer acres tilled per day - Harsh ride - Impact breaks | Under inflation may cause: - Sidewall damage - Poor ride - Bead un-seating - Fast wear |
| --- | --- |
| Proper inflation will optimize: - Traction - Compaction (reduce) - Wear - Fuel efficiency - Acres worked - Ride |  |

# Equipment:

- Tractor

- Truck scale (optional)

- Accurate tire pressure gauge

# Procedure:

Record your data in the data table.

- Determine the tire size and maximum tire pressure from the sidewall of the tires and record in the data table.

- Measure the current tire pressure in all of the tires and record in the data table.

- Determine the axle load for your tractor using a truck scale.   A truck scale is best. If a truck scale is not available, you can use the weights in the tractor manual or call you tractor dealer for the tractor's front and rear axle load.  Note:  You weight will differ if substantial weighting or attachments have been added to the tractor.  Include any cast or liquid weights, tanks, loaders and the implement hitch weight.*  To use a truck scale:

  - Roll the front wheels on to the scale.   Dismount the tractor.  Measure the weight.

  - Roll the rear wheels (only) on the scale.   Dismount the tractor.  Measure the weight.

- Using the load tables attached or those in the tractor manual determine the proper tire inflation.   Begin at the left side of the table and move right until you either find the load for your tire or the next higher load. Move up the column to find the correct tire pressure for your tire size.   Record the pressure in your data table.

- Set the tire inflation. **

*When using heavy 3-point hitch mounted equipment, adjust rear inflation pressures based on the weight being carried on the rear axle with the implement raised. Return to the lower rear inflation pressures when using towed implements. NOTE: For towed implements which transfer heavy vertical loads to the drawbar (i.e., laser scrapers, grain carts), weigh tractor with maximum axle load and adjust inflation accordingly.

**It is very important to use an accurate tire gauge. Checking with a gauge that is off by 2 psi can lead to serious tire durability problems when setting tires at these lower pressures. Tires pressures fluctuate with air temperatures and should always be rechecked on a weekly basis.

# Data Table:

| Front Tire Size (from tire sidewall) |  |
| --- | --- |
| Rear Tire Size (from tire sidewall) |  |
| Front Tire Maximum Pressure (from tire sidewall) |  |
| Rear Tire Maximum Pressure (from tire sidewall) |  |
| Front Tire Pressure (L/R) | / |
| Rear Tire Pressure (L/R) | / |
| Front Axle Weight |  |
| Rear Axle Weight |  |
| Recommended Front Tire Pressure |  |
| Recommended Rear Tire Pressure |  |

# Questions:

- Explain each part of the rear tire size (what the numbers and letters mean).

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

- Were the tires front axle inflated to the same pressure?

___________________________________________________________________

- Were the tires rear axle inflated to the same pressure?

___________________________________________________________________

- Were the tires on the front axle properly inflated?   If not how will this affect performance?

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

- Were the tires on the rear axle properly inflated?   If not how will this affect performance?

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

___________________________________________________________________

Video: https://www.youtube.com/watch?v=UGK4RV-7vVo

# Recommended Pressures-Front Radial Tires

| AxleLoad | 13.6R28 *** | 14.9R28 *** | 14.9R30 *** | 16.9R26 ** | 16.9R28 136A8** | 290/95R34 *** | 320/85R34 *** | 480/70R28 140A8** |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kg (lbs) | kPa(bar)(psi) | kPa(bar)(psi) | kPa(bar)(psi) | kPa(bar)(psi) | kPa(bar)(psi) | kPa(bar)(psi) | kPa(bar)(psi) | kPa(bar)(psi) |
| 1360(3000) | 40(0.4)(6) | 40(0.4)(6) | 40(0.4)(6) | 40(0.4)(6) | 40(0.4)(6) | 40(0.4)(6) | 40(0.4)(6) | 40(0.4)(6) |
| 1590(3500) | 60(0.6)(8) | 40(0.4)(6) | 40(0.4)(6) | 40(0.4)(6) | 40(0.4)(6) | 40(0.4)(6) | 40(0.4)(6) | 40(0.4)(6) |
| 1820(4000) | 70(0.7)(10) | 50(0.5)(7) | 50(0.5)(7) | 40(0.4)(6) | 40(0.4)(6) | 60(0.6)(8) | 60(0.6)(8) | 40(0.4)(6) |
| 2040(4500) | 80(0.8)(12) | 60(0.6)(9) | 60(0.6)(9) | 50(0.5)(7) | 50(0.5)(7)) | 80(0.8)(11) | 70(0.7)(10) | 40(0.4)(6) |
| 2270(5000) | 100(1.0)(15) | 80(0.8)(11) | 70(0.7)(10) | 60(0.6)(8) | 60(0.6)(8) | 90(0.9)(13) | 80(0.8)(12) | 40(0.4)(6) |
| 2500(5500) | 120(1.2)(17) | 90(0.9)(13) | 80(0.8)(12) | 70(0.7)(10) | 60(0.6)(9) | 100(1.0)(15) | 100(1.0)(14) | 50(0.5)(7) |
| 2720(6000) | 140(1.4)(20) | 100(1.0)(15) | 100(1.0)(14) | 80(0.8)(11) | 80(0.8)(11) | 120(1.2)(17) | 110(1.1)(16) | 60(0.6)(9) |
| 2950(6500) | 160(1.6)(23) | 120(1.2)(17) | 110(1.1)(16) | 90(0.9)(13) | 80(0.8)(12) | 130(1.3)(19) | 120(1.2)(18) | 70(0.7)(10) |
| 3180(7000) | 180(1.8)(26) | 140(1.4)(20) | 130(1.3)(19) | 100(1.0)(15) | 100(1.0)(14) | 140(1.4)(21) | 140(1.4)(20) | 80(0.8)(11) |
| 3410(7500) | 210(2.1)(30) | 150(1.5)(22) | 140(1.4)(21) | 120(1.2)(17) | 110(1.1)(16) | 160(1.6)(23) | 150(1.5)(22) | 90(0.9)(13) |
| 3630(8000) | 210(2.1)(30) | 170(1.7)(25) | 160(1.6)(23) | 130(1.3)(19) | 120(1.2)(18) | 180(1.8)(26) | 170(1.7)(24) | 100(1.0)(14) |
| 3860(8500) | - | 190(1.9)(27 | 180(1.8)(26) | 140(1.4)(21) | 140(1.4)(20) | 190(1.9)(28) | 180(1.8)(26) | 110(1.1)(16) |
| 4090(9000) | - | 210(2.1)(30) | 190(1.9)(28) | 160(1.6)(23) | 150(1.5)(22) | 210(2.1)(30) | 190(1.9)(28) | 120(1.2)(17) |
| 4310(9500) | - | 210(2.1)(30) | 210(2.1)(30) | 170(1.7)(24) | 160(1.6)(23) | 210(2.1)(30) | 210(2.1)(30) | 130(1.3)(19) |
| 4540(10000) | - | 210(2.1)(30) | 210(2.1)(30) | 170(1.7)(24) | 160(1.6)(23) | - | 210(2.1)(30) | 140(1.4)(20) |
| 4760(10500) | - | - | - | 170(1.7)(24) | 160(1.6)(23) | - | - | 140(1.4)(21) |
| 4990(11000) | - | - | - | - | - | - | - | 160(1.6)(23) |
| 5440(12000) | - | - | - | - | - | - | - | 160(1.6)(23) |
| 5900(13000) | - | - | - | - | - | - | - | - |
| 6350(14000) | - | - | - | - | - | - | - | - |

# Recommended Pressures-Rear Radial Tires

|  | SINGLES |  | DUALS |  |
| --- | --- | --- | --- | --- |
| Axle Load | 14.9R46 *** | 18.4R38 * | 14.9R46 *** | 18.4R38 * |
| Kg (lbs)a | kPa(bar)(psi) | kPa(bar)(psi) | kPa(bar)(psi) | kPa(bar)(psi) |
| 3180(7000) | 90(0.9)(13) | 60(0.6)(8) | 70(0.7)(10) | 40(0.4)(6) |
| 3400(7500) | 100(1.0)(14) | 60(0.6)(9) | 70(0.7)(10) | 40(0.4)(6) |
| 3630(8000) | 110(1.1)(16) | 70(0.7)(10) | 70(0.7)(10) | 40(0.4)(6) |
| 3860(8500) | 120(1.2)(18) | 80(0.8)(11) | 70(0.7)(10) | 40(0.4)(6) |
| 4090(9000) | 130(1.3)(19) | 80(0.8)(12) | 60(0.6)(9) | 40(0.4)(6) |
| 4310(9500) | 140(1.4)(21) | 90(0.9)(13) | 70(0.7)(10) | 40(0.4)(6) |
| 4540(10000) | 160(1.6)(23) | 100(1.0)(15) | 70(0.7)(10) | 40(0.4)(6) |
| 4760(10500) | 170(1.7)(25) | 100(1.0)(15) | 70(0.7)(10) | 40(0.4)(6) |
| 4990(11000) | 190(1.9)(27) | 120(1.2)(17) | 70(0.7)(10) | 50(0.5)(7) |
| 5220(11500) | 210(2.1)(30) | 120(1.2)(17) | 80(0.8)(11) | 50(0.5)(7) |
| 5440(12000) | 210(2.1)(30) | 120(1.2)(18) | 80(0.8)(12) | 60(0.6)(8) |
| 5670(12500) | - | - | 90(0.9)(13) | 60(0.6)(8) |
| 5900(13000) |  |  | 100(1.0)(14) | 60(0.6)(9) |
| 6120(13500) | - | - | 100(1.0)(15) | 60(0.6)(9) |
| 6350(14000) | - | - | 110(1.1)(16) | 70(0.7)(10) |
| 6580(14500) | - | - | 120(1.2)(17) | 70(0.7)(10) |
| 6800(15000) | - | - | 120(1.2)(18) | 80(0.8)(11) |
| 7030(15500) | - | - | 130(1.3)(19) | 80(0.8)(12) |
| 7260(16000) | - | - | 140(1.4)(20) | 80(0.8)(12) |
| 7480(16500) | - | - | 140(1.4)(21) | 90(0.9)(13) |
| 7720(17000) | - | - | 150(1.5)(22) | 100(1.0)(14) |
| 7940(17500) | - | - | 170(1.7)(24) | 100(1.0)(15) |
| 8170(18000) | - | - | 170(1.7)(24) | 100(1.0)(15) |
| 8400(18500) | - | - | 180(1.8)26 | 110(1.1)(16) |
| 8630(19000) | - | - | 190(1.9)(27) | 120(1.2)(17) |
| 8850(19500) | - | - | 190(1.9)(28) | 120(1.2)(18) |
| 9080(20000) | - | - | 200(2.0)(29) | - |
| 9310(20500) | - | - | 210(2.1)(30) | - |
| aTotal Rear Weight |  |  |  |  |

# Teaching Notes:

Since the proper tire pressure is related to load and tractor tire pressure is commonly much lower than cars and trucks tractor tires are commonly over inflated.

Almost any tractor can be used for this activity.   If you have access to a scale having the students weigh the tractor is desirable.   However this can be “given” information or students can look up in the manual.   To prepare for this activity the teacher can set the tire pressure improperly to foster discussion.  For example set to the maximum or 5 psi high.

## Student Learning Outcomes:

Students will:

- Determine the tractor tire size by reading the sidewall of the tire.

- Properly determine the tire pressure using a tire pressure gauge.

- Measure the axle load using a scale or the operator’s manual (reading specifications).

- Demonstrate how to find the proper inflation based on axle load and tire size.

## Teaching Outline:

- Describe the information on the tire sidewall including the meaning of the tire size.

- Determine what the students know about tire inflation by asking what they think it should be.

- Demonstrate how to measure the axle load (weigh) or look up.

- Demonstrate how to properly measure tire pressure and determine if liquid ballast is present.

- Demonstrate how to use the load table (attached or from the tractor manual).

- Discuss the findings of the activity.

## How to read the sidewall

TIRE SIZE

- Standard tire sizes are the easier of the two measurements to read. An example is “16.9 – 30”. Here, “16.9” indicates the width of the tire in inches and “30” indicates the tire will fit a wheel that is 30 inches in diameter. The “-” indicates the tire is bias-ply design. A standard tire size replacing the “-” with an “R” indicates the tire is radial design. To clarify, radial and bias-ply designs will be explained later on. Some standard sizes include the tire width. An example of what this would look like is “27 x 8.5 – 15” with “8.5” indicating the width of the tire in inches.

- Metric format. Sizes like “380/70R24” are metric and found on most modern tires. Firstly, “380” indicates the width of the tire in mm. Secondly, the “70” indicates the Aspect Ratio of the tire. Width multiplied by aspect ratio gives sidewall height, so (380 x 70% = 266). Therefore, sidewall height of this tire is 266mm. “R” means the tire is radial. Lastly, “24” is the rim diameter in inches.

- Tire sizes containing “R” are radial. Radial tires contain an embedded structure around the circumference of the tire. Radial treads flex independently from the sidewall. Therefore, allowing for more traction, even tread wear, and less of a footprint left in the field.

- Tire sizes containing “-” are bias-ply. Bias ply tires contain multiple overlapping layers at an angle of 30-45 degrees to the tread line. Bias ply tires are stiff, so they are less prone to puncture when navigating over rough ground.

LOAD INDEX NUMBER AND SPEED RATING

The load index is the two or three digit code that indicates the load carrying capacity of the tire. Maximum load goes hand in hand with the speed states by the speed symbol. Together these numbers can be considered a variable performance rating.

- Load Index - Load carrying capacity of the tire.

- Load Index Values - Range from 50-209.

- International Load Index Chart

- Speed Rating - Maximum speed for tire.

- A Rates Tires - Slower speeds.

- G Rated Tires - Faster speeds.

Note:  Exceeding the maximum load may cause the tire to fail or come off the rim.   For dual wheel tractors removing a set of tires from the axle may then overload the remaining tires.

DATE CODE

Date Codes tell you where your tire was made as well as what week of what year it was manufactured.

- Code for manufacturing location

- First two numbers - This is the week your tire was made.

- Last two numbers - This is the year your tire was made.

- "0819" 0 Indicates a tire manufactured on the 8th week of 2019.

MAXIMUM INFLATION PRESSURE

The maximum inflation pressure is a found as part of the safety warning.   Exceeding this pressure may cause the tire bead to come off the rim.

Based on: https://commercial.firestone.com/en-us/agriculture/resources/technical-bulletins/maximize-tractive-performance#