# Getting to Know Tractors

Name:_____________________

Date: _____________________

## Description

While all tractors share similar controls and service requirements; each is different.   The purpose of this exercise is to become familiar with:

- Tractor controls

- Daily Check Items including safety items.\

- Differentiate between routine maintenance items and those that can compromise operator safety.

- The operator’s manual

## Tools and Materials

Three tractors and manuals.  Shop rags. Tire pressure gauge.

## Pre-start Daily Check Items (generic list)

- The list below will work on most tractors when an operator’s manual is not available.  Determine which items you think are safety related or preventive maintenance of the machine. Indicate which with a check.

|  | - Maintenance | - Safety Related |
| --- | --- | --- |
| - Engine Oil |  |  |
| - Hydraulic/Transmission Oil |  |  |
| - Coolant Level |  |  |
| - Fuel Level |  |  |
| - Air cleaner valve & pre-cleaner |  |  |
| - Radiator Screen Clear |  |  |
| - Drain the Water Separator |  |  |
| - Tire Inflation and condition |  |  |
| - Safety shields in place |  |  |
| - Seat belt (with ROPS) |  |  |
| - L and R Brakes have same travel, locked for road travel |  |  |
| - Implement hitch pins in place |  |  |
| - PTO secure and guards in place |  |  |
| - Steering connections tight |  |  |
| - Walk around – No leaks, damage, loose or missing parts |  |  |
| - Instrument Panel (key on) |  |  |

- For this activity you will use the operator’s manual to determine the pre-start checks and locate common controls.

- Sample Diagram (combined pre-start check and controls)

Steering Wheel

Seat

Tire

Oil

3 point control

Fuel

- At the completion of the activity, students should be able to perform the pre-start check of each tractor on their own and identify common tractor controls.  Diagrams should look similar to the sample above.   Each student should check each item.  We will review and discuss all tractors when all groups are finished.

- Tractor Make ___________   Model __________ Type of drive (circle): 2WD, MFWD, 4WD

Using the operator’s manual, perform the daily check (10 hour service) of the tractor.  List any discrepancies.

_______________________________________________________________

_______________________________________________________________

_______________________________________________________________

Locate and neatly label the items listed in the operator’s manual for the daily check on the diagram below (see example on the first page). Show engine oil, hydraulic oil, coolant even if they are not part of the daily check.

Using the operator’s manual as a guide, identify and label the location of the following controls on the diagram below:  If the tractor does not have the control, then note this.

Clutch

Brakes

Differential Lock

PTO Control Lever

Gear Shift

Hi/Low or Range Lever

Forward/Reverse Lever

Throttle (foot and hand)

3 point controls

- Tractor Make ___________   Model __________ Type of drive (circle): 2WD, MFWD, 4WD

Using the operator’s manual, perform the daily check (10 hour service) of the tractor.  List any discrepancies.

_______________________________________________________________

_______________________________________________________________

_______________________________________________________________

Locate and neatly label the items listed in the operator’s manual for the daily check on the diagram below (see example on the first page). Show engine oil, hydraulic oil, coolant even if they are not part of the daily check.

Using the operator’s manual as a guide, identify and label the location of the following controls on the diagram below:  If the tractor does not have the control, then note this.

Clutch

Brakes

Differential Lock

PTO Control Lever

Gear Shift

Hi/Low or Range Lever

Forward/Reverse Lever

Throttle (foot and hand)

3 point controls

- Tractor Make ___________   Model __________ Type of drive (circle): 2WD, MFWD, 4WD

Using the operator’s manual, perform the daily check (10 hour service) of the tractor.  List any discrepancies.

_______________________________________________________________

_______________________________________________________________

_______________________________________________________________

Locate and neatly label the items listed in the operator’s manual for the daily check on the diagram below (see example on the first page). Show engine oil, hydraulic oil, coolant even if they are not part of the daily check.

Using the operator’s manual as a guide, identify and label the location of the following controls on the diagram below:  If the tractor does not have the control, then note this.

Clutch

Brakes

Differential Lock

PTO Control Lever

Gear Shift

Hi/Low or Range Lever

Forward/Reverse Lever

Throttle (foot and hand)

3 point controls

- Using the specifications sheet in the provided manual determine the following:

| Tractor Make/Model: |  |
| --- | --- |
| Advertised PTO Horsepower (hp) |  |
| Standard hydraulic pump output (GPM) |  |
| Category of 3 point hitch |  |
| Fuel Capacity (gal) |  |
| PTO speeds (speed at engine RPM) |  |
| Shipping weight (2WD) |  |

### Key Points:

At the conclusion of the activity you should be able to:

- Read a manual to determine basic specifications and daily servicing.

- Preform the daily service on a tractor.

- Understand the function of common controls found on tractors.

## Notes:_________________________________________________________________________________________ ______________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

## Getting to Know Tractors Worksheet

Name: ________________________

Date: ________________________

- If a tractor has a high/low range selector and a gear shift with 4 speeds how many gears would you expect the tractor to have?  ___________

- Why is it important to look for loose parts on a tractor?

- How can you easily tell if a tractor has Mechanical Front Wheel Drive (MFWD)?

- How can you determine the correct tire inflation pressure?

- What controls are used to vary the ground speed of the tractor?

- Explain the difference between a category 1 and a category 2 three point hitch.

- When is a seatbelt required on a tractor?

# Getting to Know Tractors Teachers Notes:

## Agricultural Standards:

6.0	Health and Safety. Students understand health and safety policies, procedures, regulations, and practices, including the use of equipment and handling of hazardous materials:

B1.0 Students understand personal and group safety:

B1.1 Practice the rules for personal and group safety while working in an agricultural mechanics environment.

B1.2 Know the relationship between accepted shop management procedures and a safe working environment.

B11.0 Understand the principles and applications of various engines and machinery used in agriculture.

B11.1 Identify common agricultural machinery and implements.

B11.2 Calibrate, operate, and maintain equipment safely and efficiently.

## Objectives:

By properly completing this activity, students will be able to:

Understand the importance of the pre-start check

How to perform the pre-start check

How to use an operator’s manual

Identify the controls commonly found on tractors

Demonstrate how to properly get on and off a tractor

## Alternative Tools/Methods/Materials:

Try to get three different tractors (different sizes, drive types, etc.).   Note:  Often a local dealer can provide the tractors.   Activity can really be done with any number of tractors, three seems to work well to get a mix of tractors, and manageable group sizes.  ATVs or other engine driven machinery can be included in this activity (diagram will need to altered).  The generic checklist can be laminated and used by student during operations.

## Safety Review:

How to properly get on and off a tractor

What items on the checklist can affect the operator’s safety.

## Activity Time:

Recommended lesson is to do an introductory demonstration.  Divide the students into groups (1 group per tractor).  Have the students rotate between the tractors.  Gather the class and have each group report on the last tractor they used.

| Demonstration: | 20 minutes |
| --- | --- |
| Activity | 3 hours |

Demonstration Notes

- Demonstrate how to find the daily check in the manual.

- Demonstrate how to check oil levels with a dip stick.

- Explain how tire to read the tire size.

- Explain how tire pressure is determined.

- Demonstrate how to properly get on and off a tractor.

- Demonstrate how to determine the difference between diesel fuel and gasoline.

- Explain when seatbelts are required and when they should not be used.

- Identify common parts of the tractor (ex. 3 point hitch).

| # Tractor Driving Daily Checklist |  |  | - Tractor Driving Daily Checklist |  |
| --- | --- | --- | --- | --- |
| Pre-Start: Engine Oil Hydraulic/Transmission Oil Coolant Level Fuel Level Air cleaner valve & pre-cleaner Radiator Screen Clear Drain the Water Separator Tire Inflation and condition Safety shields in place Seat belt (with ROPS) L and R Brakes have same travel, locked for road travel Implement hitch pins in place PTO secure and guards in place Steering connections tight Walk around – No leaks, damage, loose or missing parts Instrument Panel (key on) | Personal: I am prepared to drive safely Wearing snug fitting clothes Long hair under cap Boots or heavy shoes Safety glasses/hearing protection After starting: Oil Pressure Instrument Panel Hydraulic response Brakes Test Steering Operating: Coolant Temperature Oil Pressure Fuel Level Hydraulic Response 3 L’s – Listen, Look, Live |  | Pre-Start: Engine Oil Hydraulic/Transmission Oil Coolant Level Fuel Level Air cleaner valve & pre-cleaner Radiator Screen Clear Drain the Water Separator Tire Inflation and condition Safety shields in place Seat belt (with ROPS) L and R Brakes have same travel, locked for road travel Implement hitch pins in place PTO secure and guards in place Steering connections tight Walk around – No leaks, damage, loose or missing parts Instrument Panel (key on) | Personal: I am prepared to drive safely Wearing snug fitting clothes Long hair under cap Boots or heavy shoes Safety glasses/hearing protection After starting: Oil Pressure Instrument Panel Hydraulic response Brakes Test Steering Operating: Coolant Temperature Oil Pressure Fuel Level Hydraulic Response 3 L’s – Listen, Look, Live |
| - Tractor Driving Daily Checklist |  |  | - Tractor Driving Daily Checklist |  |
| Pre-Start: Engine Oil Hydraulic/Transmission Oil Coolant Level Fuel Level Air cleaner valve & pre-cleaner Radiator Screen Clear Drain the Water Separator Tire Inflation and condition Safety shields in place Seat belt (with ROPS) L and R Brakes have same travel, locked for road travel Implement hitch pins in place PTO secure and guards in place Steering connections tight Walk around – No leaks, damage, loose or missing parts Instrument Panel (key on) | Personal: I am prepared to drive safely Wearing snug fitting clothes Long hair under cap Boots or heavy shoes Safety glasses/hearing protection After starting: Oil Pressure Instrument Panel Hydraulic response Brakes Test Steering Operating: Coolant Temperature Oil Pressure Fuel Level Hydraulic Response 3 L’s – Listen, Look, Live |  | Pre-Start: Engine Oil Hydraulic/Transmission Oil Coolant Level Fuel Level Air cleaner valve & pre-cleaner Radiator Screen Clear Drain the Water Separator Tire Inflation and condition Safety shields in place Seat belt (with ROPS) L and R Brakes have same travel, locked for road travel Implement hitch pins in place PTO secure and guards in place Steering connections tight Walk around – No leaks, damage, loose or missing parts Instrument Panel (key on) | Personal: I am prepared to drive safely Wearing snug fitting clothes Long hair under cap Boots or heavy shoes Safety glasses/hearing protection After starting: Oil Pressure Instrument Panel Hydraulic response Brakes Test Steering Operating: Coolant Temperature Oil Pressure Fuel Level Hydraulic Response 3 L’s – Listen, Look, Live |