Name: __________________

Period: __________________

# Right Angle Layout

Most building construction projects require the layout to be square and either level or at a specific grade.  Laying out a square corner can be easily done with two tapes and stakes.   Square corners are commonly needed for building foundations and concrete slabs.   The method described below uses a “3-4-5” right triangle to construct the square corner.

# Materials

Wooden stakes (12”)

# Tools

2 - 25, 50, 100 tapes (length depends on the size of the project)

Engineer’s or similar hammer.

# Procedure

- Determine the size of the building from plans.

- Establish one side of the building with two stakes.   Locate the stakes so one corner will be on the inside of the building. (See Diagram for 36’x43’ building below.  Stakes 1 & 2).  The stakes should be spaced the exact distance of the building dimensions.

- To set a square corner you will need to establish a “3-4-5” triangle.   This is a ratio so you can choose any combination of numbers as long as the ratio is maintained.   Some examples are:

| 3 | 4 | 5 |
| --- | --- | --- |
| 15 | 20 | 25 |
| 18 | 24 | 30 |
| 30 | 40 | 50 |
| 300- | 400 | 500 |

- Consider the line between the two stakes the “base” of the triangle.   Measure from one of the stakes out to the “4” side distance and place a stake (stake 3 in the diagram).

- Now using two tapes measure up the “3” side to the desired distance and measure from the “4” stake the “5” distance (the hypotenuse of the triangle).   Set a stake (stake 4 in the diagram) where the tapes cross.   This establishes the right angle (square).

- Using a tape extend the line from the square stake to the corner of the building and set a stake (stake 5 in the diagram).  This is the third corner of the building.

- Now again using two tapes and the building dimensions measure from the two corner stakes to find the final stake location (stake 6 in the diagram).

- Stakes 3 and 4 can now be removed.

Sample layout of a 36’x43’ building.

# Questions

- If you measure 75’ on one side of the triangle and 100’ on another what will the length be on the third (long) side if it is a right triangle?

________________________________________________________

- What is the hypotenuse?

________________________________________________________

- Describe how you can check your final layout for square.

________________________________________________________

________________________________________________________

________________________________________________________

________________________________________________________

________________________________________________________

# Teaching Notes

- Any fairly level grass area will work well for this activity.  It is easier to put in stakes if the ground is soft.

- Teams of three work well (min of two).

- Alter the building size as needed.   Student groups can use different dimensions.  The procedure is the same.