Name: __________________

Period: __________________

# Instrument (Level) Setup

To get accurate readings the instrument or level must be correctly setup on the tripod.  If the instrument is not level any readings taken will not be accurate.   Every time the instrument is moved you must perform this procedure.

# Materials

Handouts

Cable ties

# Tools

Level

Tripod

Rod (2)

# Procedure:

## Tripod and Instrument Attachment

- When setting up the tripod, make sure the three tripod points are firmly into the ground and the top of the tripod head is as level as possible.  The legs should be about 3’ apart.

- Adjust the height of the tripod to a comfortable viewing height, and secure the retractable legs.  The head should be close to level.

- Attach the instrument to the tripod head with the center screw and tighten securely.

## Leveling of the Instrument (3 Screw)

- Mount the instrument on the tripod and line up the telescope vial in position (a) as shown.

- Then grasp screws A & B so that both thumbs are moving in opposite directions, either toward each other or away from each other.

- Note that the bubble moves in the same direction as your left thumb.

- Keep about half the foot screws length engaged.

- When the bubble is centered in position (a), turn the instrument and observe the vial in position (b).

- Now center the bubble in position (b) using only screw C.

- Check to see that the instrument is level by rotating to random positions.  The level bubble should remain centered.   If not repeat the procedure.

## Leveling of the Instrument (4 Screw)

- Mount the instrument on the tripod and line up the telescope inline with two opposite screws (a).

- Adjust the screws that are aligned to the telescope.  Grasp screws the two screws so that both thumbs are moving in opposite directions, either toward each other or away from each other.

- Note that the bubble moves in the same direction as your left thumb.  Adjust until the bubble is centered.

- Rotate the telescope 90 degrees so it aligns with the other two screws (b).  Repeat steps 2 and 3.

- Rotate the telescope back to the original position and adjust as necessary.

- Check to see that the instrument is level by rotating to random positions.  The level bubble should remain centered.   If not repeat the procedure.

## Aiming and Focusing

- Rotate the instrument by hand to aim the telescope to the far object.

- Rotate the focusing knob until the object can be observed clearly.

- Always focus the telescope before attempting to read the rod.

## Cross-hair Focus

- Before beginning using the instrument you must be sure that the cross-hairs are sharply focused.

- Adjust the eye piece until the cross-hairs are sharply focused.  Once focused the cross-hairs should remain in focus for all readings.

- NOTE:  If the operator changes readjust the cross-hair focus as their eye-sight may be different.

## Practice Reading

- Point the telescope at rod #1

- Focus the telescope

- Read the rod and record your reading.

- Repeat for rod #2.

# Questions

- What type of level are you using? _____________________________

- What is the proper height of the telescope?

________________________________________________________

- When should the cross-hair focus be readjusted?

________________________________________________________

- When will the telescope focus need to be adjusted?

________________________________________________________

- What are the graduations (ex.  Feet, inches or Feet and 100ths of feet) is the rod you are using?

________________________________________________________

- If you take rod readings of 5’ 4” and 5’ 6” which point is lower?

________________________________________________________

- Rod #1 Reading: 	________

- Rod #2 Reading: 	________

- Difference (subtract reading #2 from reading #1): 	________

# Reading the Rod

Reading a rod takes a bit of practice.  The reading is taken at edge of the mark not the middle.   The diagram below shows a Philadelphia Rod (feet and hundredths of feet).

# Teaching Notes

- Preface the activity with a lecture on the setup procedure and rod reading (or videos).

- Dirt or turf works best.  If you need to setup on asphalt create a stand for the tripod (below).

- Setup two rods in fixed positions so students can practice focusing and reading.  Set at different distances to force focusing pratice.  Rods can be fastened with a cable tie to a fence post or Tpost.    Clearly label as #1 and #2.

- Multiple levels can be used.   Note: Different levels will have different readings as their height of instrument will be different.  However, you can check reading by computing the difference in elevation between the two rods.   If the instrument is not moved and is properly leveled, then all students should have same number.

Tripod stand

Build from 1x2 lumber.   Attach pieces with one screw so they can be folded for storage.   Drill ¾” holes in the ends for the tripod points.

# Rod Labels

Cut apart and tape to each rod.

#1

#2