Name: __________________

Period: __________________

# Setting Forms to Grade

Concrete forms must be set level for buildings and with a grade or slope when drainage is desired.   The process is the same.   If building level forms then the elevation at all locations along the forms will be the same.   If setting forms for a driveway, hog pen, or similar project then a grade is calculated based on the “fall” in inches over the distance of the concrete slab.   The fall is usually given as inches per foot.  Note that we don’t care about the actual elevation of the slab only the relative difference.

# Materials:

4 steel form stakes

2 – 2x4x12’

2 – 2x4x16’

16d duplex nails or 3” screws

8d duplex nails or 1 5/8” screws

# Tools:

Level and Rod

Claw Hammer

Engineer’s Hammer

25’ steel tape

Drill/Driver if using screws

# Computing the “Fall”

We desire a slope or fall of the concrete slab to be ¼” per foot for drainage.   The slab is 16’ long.  To determine the fall multiply the slope by the distance.  In this case:

¼” / Foot X 16’ = 4”

# Procedure:

- Assemble a 12’x16’ form (interior dimensions will be slightly less).

- Layout the form on a patch of grass or other area (irrigated turf makes driving stakes easier).

- On the 12 foot side place two stakes on the outside of the form, near the corner (A & B), and nail or screw to the form about 6” off the ground.  This will be the “top” of the slab.

- Using the tape measure diagonally (A-C, B-D) across the form and adjust the form until the measurements are equal.  (The form is now square.)

- Place stakes outside of form on the side opposite the first stakes (C&D).

- Setup up the level instrument near the forms and adjust it to level.

- Level the first (top) side.  Take a measurement with the level in one corner and then a second in the other corner.   Adjust the stakes until the readings are the same.

- Now go to a lower corner and adjust the form on the stake until the reading is 4” (our fall) greater than the top.

- Repeat with at the other lower corner.

- Now double check square and all of the readings.  Adjust as necessary.

Optional:

Computer the number of yards on concrete required.  Assume the slab is 4” thick.   Round up to the nearest ½ cubic yard.   Useful information:  27 cubic feet=2 cubic yard.

A

B

D

C

# Questions

- What is the fall you will be using for this slab?

________________________________________________________

- What is the reading of the rod shown to the right?

________________________________________________________

- Which fall is steeper (slopes more), 1/8”/foot or ¼”/foot?

________________________________________________________

- If you take rod readings of 5’ 4” and 5’ 6” which point is lower?

________________________________________________________

- The top of a 20’ driveway has a rod reading of 5’ 4.25” and the bottom has a rod reading of 5’ 6.75” what is the slope in inches/foot?

________________________________________________________

- Why is it important to not move the level between readings?

________________________________________________________

________________________________________________________

________________________________________________________

# Teaching Notes

- If kept straight forms can be reused.   2x6 forms can be used.

- Stakes can be made from 1 ½” x ¼” steel or purchased.   Flat stakes are easier to use than round stakes.

- Students can work in smaller teams if you have more than one set of forms and levels.

- You can use different types of levels.

- Students can also measure existing forms (or slabs) and compute the slope.

- This activity is easily connected to a concrete unit.

- The activity is designed for use with a builder’s rod (feet and inches).   A 1” steel tape on a board will work as well.   If using a rod in feet and hundreds of feet convert the fall.  ¼”/foot = .02 feet/foot.