Name: __________________

Period: __________________

# Differential Leveling

Profiles are used to determine slopes for irrigation, drainage, and building layout.  In this exercise you will measure elevations using a level then graph the profile.

.

# Materials:

form stakes

graph paper

Magic marker

# Tools:

Level and Rod

Claw Hammer

Engineer’s Hammer

100’ steel tape

Clipboard

Pencil

# Procedure:

- Help setup the profile as directed by the instructor. Use the tape, locate, mark, and measure 20 points between the ends of the profile.  Label stakes (21) starting with 0.

- Record these distances (stations) in the table below.

- Make a sketch in the space below.

- Setup up the level mid-way between the ends of the profile.  Review how to read the rod (below).

- Work in a small teams.  Rotate the using the rod and the level during your elevation measurements.  Circle the stations on your data sheet where you read the rod.

- Place the rod on the benchmark (BM) and record your reading.  Calculate the Height of Instrument (HI) and record on the data sheet.

- Measure the Foresights (FS) of each of the remaining points and enter on your data sheet. Calculate the elevation of each point.

- As a final reading re-read the sighting to the

- Individually, use graph paper plot the elevations.  Horizontal scale will be about 1”=20’, vertical scale should be about 2”=1’.  Label the points and the graph.  (See example).

- Label the high point(s).

## Sketch:

Sketch of the area locating the course and nearby buildings and roads.  Indicate the location of the benchmark.  Show the location of your instrument.

## Profile Graph Example:

## Data Sheet:

| Station Number | Distance (ft) | B.S. | H.I. | F.S. | Elevation (ft) | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| BM | ----- |  |  |  | 10.00 | BM assume to be 10’. |
| 0 |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |

# Questions

- If you take rod readings of 5’ 4” and 5’ 6” which point is lower?

- Does it matter in determining grade that the benchmark elevation was arbitrarily 10’?  Why?

- What is the fall you will be using for this slab?

________________________________________________________

- What is the reading of the rod shown to the right?

________________________________________________________

- Which fall is steeper (slopes more), 1/8”/foot or ¼”/foot?

________________________________________________________

________________________________________________________

- What is the general grade of the profile between station 0 and station 20?  Uphill or downhill?

________________________________________________________

- What was the average elevation of the stations?   (Total the elevations and divide by21.)

________________________________________________________

________________________________________________________

________________________________________________________

# Teaching Notes

- Locate a point for a benchmark.   A curb or other concrete structure works well.   Mark the point with paint.  The location can be reused for future activities.

- Locate a 200’ profile.   Some up and down is OK.   Look for a position for the level that will have a clear view of the BM and all stations.

- Layout a row of stakes 10’ feet apart.  If you have a 300’ tape the easy way to do this is stake station 0 then stretch the tape straight and place stakes every 10’.

- Label the station number on the stake before setting it.  NOTE:  store stakes in a bucket and reuse.

- Students can work in smaller teams if you have more than one set of instruments.

- You can use different types of levels.

- A spreadsheet is available to make a key.

- If multiple instruments are used the HI will vary as will the FS.   However the elevations should be the same for all instruments.