Name: __________________

Period: __________________

# Drawing a Topo Map

Once data has been measured.

# Materials:

Graph Paper 5 sq/in

# Tools:

Pencil

# Data:

| Station Number | B.S. | H.I. | F.S. | Elevation (ft) | Comments |
| --- | --- | --- | --- | --- | --- |
| BM | 5.23 | 105.23 |  | 100 | Bench Mark |
| A1 |  |  | 11.93 | 93.30 |  |
| A2 |  |  | 11.53 | 93.70 |  |
| A3 |  |  | 7.53 | 97.70 |  |
| A4 |  |  | 9.23 | 96.00 |  |
| A5 |  |  | 7.93 | 97.30 |  |
| A6 |  |  | 7.43 | 97.80 |  |
| B1 |  |  | 10.73 | 94.50 |  |
| B2 |  |  | 9.73 | 95.50 |  |
| B3 |  |  | 9.13 | 96.10 |  |
| B4 |  |  | 8.53 | 96.70 |  |
| B5 |  |  | 7.53 | 97.70 |  |
| B6 |  |  | 7.43 | 97.80 |  |
| C1 |  |  | 9.63 | 95.60 |  |
| C2 |  |  | 8.83 | 96.40 |  |
| C3 |  |  | 8.33 | 96.90 |  |
| C4 |  |  | 7.03 | 98.20 |  |
| C5 |  |  | 5.73 | 99.50 |  |
| C6 |  |  | 6.83 | 98.40 |  |
| D1 |  |  | 8.53 | 96.70 |  |
| D2 |  |  | 7.83 | 97.40 |  |
| D3 |  |  | 6.73 | 98.50 |  |
| D4 |  |  | 5.23 | 100.00 |  |
| D5 |  |  | 5.73 | 99.50 |  |
| D6 |  |  | 6.93 | 98.30 |  |
| E1 |  |  | 7.83 | 97.40 |  |
| E2 |  |  | 6.93 | 98.30 |  |
| E3 |  |  | 5.23 | 100.00 |  |
| E4 |  |  | 6.33 | 98.90 |  |
| E5 |  |  | 7.63 | 97.60 |  |
| E6 |  |  | 7.23 | 98.00 |  |
| F1 |  |  | 8.53 | 96.70 |  |
| F2 |  |  | 8.03 | 97.20 |  |
| F3 |  |  | 7.73 | 97.50 |  |
| F4 |  |  | 8.13 | 97.10 |  |
| F5 |  |  | 11.43 | 93.80 |  |
| F6 |  |  | 9.03 | 96.20 |  |

The elevations (ft) below were measured on the field at 100’ intervals.   Draw even 1 foot contour lines for this field.

# Procedure:

- Plot the data as shown below on your graph paper.   The grid can use 1’”=100 scale.

| Row/Col | A | B | C | D | E | F | G |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 93.3 | 94.5 | 95.6 | 96.7 | 97.4 | 97.3 | 96.7 |  |  |
| 2 | 93.7 | 95.5 | 96.4 | 97.4 | 98.3 | 97.8 | 97.2 |  |  |
| 3 | 94.7 | 96.1 | 96.9 | 98.5 | 100.0 | 93.5 | 97.5 |  |  |
| 4 | 96.0 | 96.7 | 98.2 | 100.0 | 98.9 | 97.9 | 97.1 |  |  |
| 5 | 97.3 | 97.7 | 99.5 | 99.5 | 97.6 | 95.6 | 93.8 |  |  |
| 6 | 97.8 | 98.4 | 98.4 | 98.3 | 98.0 | 96.9 | 96.2 |  |  |

- In between each data point plot where the even elevation would be.  For example if two adjacent points are 95.5 and 96.5 then 95.0 would be halfway in between.  If the two points are 93.3 and 93.7 then there is no whole foot between them.   If the adjacent points are 93.7 and 95.5 then there are two whole numbers (94 and 95) between them.

- Connect the marks of the same elevation with a smooth line.

- Label each line with the elevation.

- Put your name on the graph paper and hand in with this activity.

# Questions

- Where is the high point in the field (row/column)?

________________________________________________________

- Where is the low point in the field (row/column)?

________________________________________________________

- If each data point represents a 100’ square how big is the area in acres?

________________________________________________________

- What is the average elevation of the field?

________________________________________________________

- Plot the profile of row ____ below. Elevation on the vertical axis and columns (A-G) on the horizontal axis.  Be sure to label each axis.

Useful Information:

1 acre = 43560 square feet

# Teaching Notes