Name: __________________

Period: __________________

# Surveying Unit Exam

Instructions: Circle the correct answer.  A calculator will be useful.  See useful information at the end of the exam.

- A location of known elevation is called a__________.

  - Benchmark

  - Backsight

  - Height of Instrument

  - Foresight

- The calculated elevation of the level called the __________.

  - Benchmark

  - Backsight

  - Height of Instrument

  - Foresight

- Philadelphia or Engineer’s rod us graduated in ________.

  - Feet, Inches, 10ths of inches

  - Feet, Inches, 8ths of inches

  - Feet, 100ths of feet

  - Feet, 8ths of feet

- A builder’s rod us graduated in ________.

  - Feet, Inches, 10ths of inches

  - Feet, Inches, 8ths of inches

  - Feet, 100ths of feet

  - Feet, 8ths of feet

- To check square on any rectangular (or square) shape you can __________.

  - Measure the sides

  - Measure a diagonal

  - Compare diagonal measurements

  - Measure one side and the diagonal.

- A right triangle can be constructed using sides with the following ratio ____.

  - 1,2,3

  - 2,4,6

  - 3, 6, 9

  - 3,4,5

- A sighing to a point of known elevation is called a__________.

  - Benchmark

  - Backsight

  - Height of Instrument

  - Foresight

- A sighing to a point of unknown elevation is called a__________.

  - Benchmark

  - Backsight

  - Height of Instrument

  - Foresight

- If you pace is 2.5’ and you pace off 167 paces between two buildings what is the approximate distance between the buildings?

  - 417.5’

  - 167’

  - 66.8’

  - 2.5’

- If properly setup, the bubble on a level will remain centered in any position.

  - True

  - False

- The cross hair focus of the level does not need to be adjusted before using the level.

  - True

  - False

- To focus the level you should rotate the eyepiece.

  - True

  - False

- The piece of equipment used to support a level is called a __________.

  - Rod

  - Tripod

  - Stand

  - Bench

- If a sidewalk drops 5” in 20’ what is the grade?

  - 1/8”/foot

  - ¼”/foot

  - ½”/foot

  - ¾”/foot

- To check a level building pad form with a level you should find readings at the top of the forms to be:

  - The same as the HI

  - All equal

  - 10’

  - Variable by location of the reading

- A slab for a pig barn is to be poured with a grade of 3/8” per foot.  If the slab is 16’ long how much fall should the form have?

  - 3/8”

  - 4”

  - 5”

  - 6”

- A level can measure horizontal angles.

  - True

  - False

- After setting forms for barn building pad you calculate that you need 400 cubic feet of concrete.  How many cubic yards is this (round up)?

  - 15

  - 27

  - 400

  - 10800

- What is the reading at “B”?

A

B

C

  - 3’ 9 ½”

  - 2’ 9 ½”

  - 3’ 10 ½”

  - 3’ 10 3/8”

- You measure a rectangular field to be 1300’ x 650’.   How many acres is the field?

  - 1300

  - 845000

  - 19.4

  - 43560

- In differential leveling the first reading you must take to establish the HI is:

  - A foresight

  - A backsight

  - A benchmark

  - An angle

- In an orchard you wish to determine the row spacing.   You pace the distance between 5 rows of trees as 64’.   What is the tree row spacing (think about it..)

  - 64’

  - 16’

  - 12.8’

  - 2.5’

- Topographic lines are lines of

  - Slope

  - Grade

  - Equal elevation

  - Fall

- When reading a builder’s rod the greater the reading the lower the station elevation.

  - True

  - False

- What is the rod reading at “C”?

A

B

C

  - 5 ½’

  - 5.50’

  - 5.64’

  - 6.76’

Useful Information:

1 acre=43560 square feet

1 cubic yard=27 cubic feet