Name: __________________

Period: __________________

# Survey Tools

- Tools Used in Surveying

- Distance Measuring Tools

- Level and Angle Measuring Tools

  - Survey Instruments

    - Levels –

    - Transits –

    - Total Stations –

- Tools

  - Elevation Rods

    - Philadelphia -

    - Builder’s Rod -

    - Direct Elevation -

  - Laser Levels

- Rotating laser set level or to a slope

- Uses a special target (laser detector)

- Other Tools

  - Plumb Bob

  - Surveyor’s Tape

  - Hand Level

  - Tripod -

  - Rod Targets –

Tool ID – Fill in the empty boxes

| Name | Description | Photo |
| --- | --- | --- |
| Chaining Pins | Used to mark points of distance on the ground |  |
|  | Common lengths are 50, 100, and 300’. Not as accurate as a chain. |  |
|  | A steel tape used for accurate measurement. The unit “chain” is 66 feet. |  |
| Measuring Wheel | Use to measure longer distances. More accurate than pacing, but less accurate than a tape. |  |
| Level |  |  |
|  | A telescope that can measure horizontal and vertical angles. |  |
| Total Station | A telescope that can measure horizontal and vertical angles as well as distance. Uses a reflector instead of a rod. Software does the trigonometric computations to compute the position of the target. |  |
|  | The level generates a level laser beam. The receiver is attached to a rod and moved up and down to intercept the beam. |  |
| RTK GPS | A highly accurate device that uses the Global Positioning System (GPS) for surveying and precision agriculture |  |
|  | The target us used when measuring long distances where the rod cannot be read through the telescope. The target is moved up and down on the rod by the rod man. |  |
|  | Attached to the rod and used to hold it plumb |  |
| Builder’s Rod |  |  |
| Philadelphia Rod |  |  |
|  | The rod has a movable tape that is adjustable, so the readings reflect the change in elevation. |  |
|  | Supports the instrument. |  |
|  | Used to locate an instrument above a point or to measure plumb (up and down) in building. |  |
|  | A handheld telescope with a level bubble. Useful for estimating slope. |  |
|  | Plastic tape used to mark stakes. |  |