Name: __________________

Period: __________________

# Level Setup

Leg Clamp

Head

Setting the Tripod

  - Adjust the legs to bring the head to a comfortable reading height and level. Lock the leg clamps.

- Attaching the level

  - Note that the instrument may be adjusted laterally to position a plumb bob over a BM (Bench Mark) if desired.

- Auto Levels and Laser Levels

  - Some auto levels have a bubble level to indicate that they are mounted close enough to level for the auto level to function.

- 3 Screw Levels

- Adjusting 3 Screw Levels

  - Then grasp screws A & B so that both thumbs are moving in opposite directions, either toward each other or away from each other.

  - Now center the bubble in position (b) using only screw C.

  - Check to see that the instrument is level by rotating to random positions.  The level bubble should remain centered.   If not repeat the procedure.

- 4 Screw Levels (Dumpy Levels)

- Adjusting 4 Screw Levels

- Adjust the screws that are aligned to the telescope.  Grasp screws the two screws so that both thumbs are moving in opposite directions, either toward each other or away from each other.

- Rotate the telescope 90 degrees so it aligns with the other two screws (b).  Repeat steps 2 and 3.

- Check to see that the instrument is level by rotating to random positions.  The level bubble should remain centered.   If not repeat the procedure.

- Aiming and Focusing

  - Always focus the telescope before attempting to read the rod.

Cross Hairs

Cross Hairs

- Cross Hair Focus

  - Place a piece of paper in front of the telescope.  Adjust the eye piece until the cross-hairs are sharply focused.  Once focused the cross-hairs should remain in focus for all readings.

  - NOTE:

- Stadia Distance Measuring

  - So, if the difference is 1 foot, the person holding the grade rod is 100 feet away from the instrument.