Name: __________________

Period: __________________

# Rod Reading

- Rods

  - Three type of rods are commonly used in agriculture:

  - Direct Elevation Rods – Feet and 100ths of feet (commonly used with laser levels)

- Using the Rod

  - The rod must be held plumb (vertical).   If the rod is slanted then the reading will be greater than the actual vertical distance.  Rod Levels can help the user hold it plumb.

- Rod Target

  - The rodman then reads and records the reading based on the position of the target.

- Reading the Rod Scale

  - Rod scales differ from tapes.  With a tape you read the marks (graduations).  With a rod the marks are wide and you read the edges.

- Numbers

  - Feet are also repeated as small numbers to aid in reading when you cannot view the whole foot.

- Markings

  - The next longest mark is ½ at the point edge.  In builder’s rods this will be ½”, In Philadelphia rods this will be .05’.

- Graduations

- Commonly even feet are written in red and inches or 10ths of feet in black letters.

A

B

C

- Builder’s Rod

  - What is the reading at “A”

  - What is the reading at “B”

  - What is the reading at “C”

A

B

C

- Philadelphia Rod

  - What is the reading at “A”

  - What is the reading at “B”

  - What is the reading at “C”

- Laser Level Receivers

  - The reading is then taken at the pointer location.

- Direct Elevation Rods

  - It reads directly (or backwards) so the lower the elevation the lower the rod reading.   No subtraction of the HI is required.