Name: __________________

Period: __________________

# Setting Grade

- Introduction

  - Common grades might be 1/8”/ foot or ¼”/foot.  For example the fall in a 16’ driveway at ¼”/foot would be:
¼”/foot x 16’ = 4”

- Procedure for small forms

A

B

D

C

- Calculate the “fall”

- Procedure for small forms

  - On the 12-foot side place two stakes on the outside of the form, near the corner (A & B), and nail or screw to the form about 6” off the ground.  This will be the “top” of the slab.

  - Now go to a lower corner and adjust the form on the stake until the reading is 4” (our fall) greater than the top.

  - Now double check square and all of the readings.  Adjust as necessary.

- Calculating Concrete Volume

12’ x 16’ x 4” ÷ 12”/foot = 64 cubic feet

  - 64 cu.ft. ÷ 27 cu.ft./cu.yd. = 2.37 cu.yd. ≈ 2.5 cu.yd.*
(* we round up for ordering)

- Batter Boards

  - Horizonal “ledger boards” are attached so the top represents the finished grade. These are rechecked with an instrument.

  - For rectangular forms the diagonals are measured to ensure string lines are square.

- Setting Grade for a Pipeline

For example:

  - Stakes can be used (marked with grade) or the pipe can be adjusted as each section is laid.

- Setting Grade for Surface Drainage

- For surfaces like parking lots or yards stakes are usually installed at regular intervals and marked as cut (C) or Fill (F).

- Cut stakes will have a distance (ex. 0.43’) from the surface to the proper cut depth.

- Marks call a crowfoot look like sideways K.  Are used as a reference to measure from. The “>” in the crowfoot points in the direction of the measurement.

- Grade Problems

  - When setting forms for building foundation if one corner reads 5’ 4 3/8” what should the other corners read?

  - A 20’ long driveway is designed to have a ¼”/foot slope.   How much fall will there be in the forms?

  - A sewer pipe from a hog barn to a pond is 100’ long.  To maintain a grade of ¼”/foot how much fall is needed?

  - In the above problem the pipe elevation at the barn is 4’ 3 ½”.  What should elevation be at the pond?