Name: __________________

Period: __________________

# Differential Leveling

- Definition and Use

- Differential leveling is the process of measuring vertical distances from a known elevation point to determine elevations of unknown points.

- Common applications of differential leveling are:

- Terms

- Bench Mark (BM) – A point of _____________ (or assigned) elevation.

- Backsight (BS) – A sighting or measurement to a point of _______________ elevation (BM).

- Height of the Instrument (HI) – The computed ___________________of the level.

- Foresight (FS) – A sighting to a point of _____________________ elevation.

- Turning Point (TP) – A foresight that is then used for a backsight when the level is moved.

- Computations

| Station | B.S. | H.I. | F.S. | Elevation |
| --- | --- | --- | --- | --- |
| BM | 5.50 | 105.50 |  | 100.00* |
| 1 |  |  | 4.50 | 101.00 |
| 2 |  |  | 3.50 | 102.00 |
| 3 |  |  | 4.00 | 101.50 |
| 4 |  |  | 4.50 | 101.00 |
| 5 |  |  | 5.00 | ________ |
| *known |  |  |  |  |

- Keeping the “book”

- Measurement are recorded in columns to locate the _________ and __________________.

- HI and Elevations are ________________.

- Leveling Procedure

- The BM elevation is known or set to an even height (ex. 100.00 feet).

- Set up and level the instrument at the approximately ______________ between the BM and turning point (TP).

  - It is generally recommended that the distance from the instrument to the benchmark not exceed 400 feet; approximately 200 feet is the usually recommended distance.

  - It is also recommended that the instrument be set up close to halfway between two benchmarks in order to cancel out leveling errors.

- Send the rodman to the BM and record the rod reading. This is what is known as a _____________ reading and is recorded under BS in the field notebook.

  - Add the BS reading to the elevation of the BM to give the height of the instrument (HI), i.e., HI = 3.02’ + 100.00’ = 103.02’.

- Leveling Procedure (Cont.)

- Move the rodman to next station and record this as a _____________________ reading in your field notebook.

  - Subtract this FS reading from the HI reading to obtain the elevation at point TP-1, i.e., 103.02’ - 3.68’ = 99.34’.

- Move the level instrument to the second level set-up location and set up and level the instrument.

- Take another rod reading from back to the last foresight (TP). Record this backsight reading (3.34’) in the field notebook.

  - Add the BS reading to the elevation of TP-1 to give the height of the instrument (HI), i.e., HI = 3.34’ + 99.34’ = 102.68’.

- Repeat the process until all stations are measured.

- Differential Leveling Process