Name: __________________

Period: __________________

# Topo Mapping and Land Leveling

- What is Topography Mapping?

  - The process of determining the station elevation remains the same.

- Examples

- Uses in Agriculture

Unlevel Field
Checks Follow Natural Contours

Land Leveling for Irrigation

  - Rice Checks

    - Checks are small ridges that hold the water and must be at a fixed elevation.

Leveled Field
Checks are Straight

- Historic Leveling Practice

  - Staking.

    - Cuts (where soil is removed), and fills (where soil is added) were computed for each stake

  - Manual Grading

  - Laser grading

Transmitter

(photo next slide)

Receiver

Laser Equipment

Receiver

Transmitter

Modern Practice

  - First the tractor or ATV with the receiver and the GPS unit will log elevations, collecting many more data points than traditional staking.

  - Third the operator of the earth moving equipment then drives around the field and the GPS adjusts the implement to make the cuts and fills.

GPS Receiver

GPS - Equipment

- GPS – Accuracy

  - Survey units use a form of GPS called Real-time Kinematic (RTK).   This requires a base station located at a known point within 6 miles of the survey location.

  - RTK is used by land surveyors, land levelers, and for precision guided agricultural equipment.

- RTK GPS Base Station

  - The base station then transmits a correction to the mobile unit in the field to allow for highly accurate measurements.

- Cut/Fill Map

- Topographic Map

  - Making a Topographic Map

    - Using a level determine elevations of the grid points

  - Establish the Grid

.

  - Create a map in a book or graph paper showing the grid points to scale.

  - Measure the Elevations

  - Transfer the elevations to your map.

  - Draw the Topo Map

  - Label the lines.

- Topographic Map - Place dots on the grid of equal elevation

- Topographic Map - Connect dots on the grid of equal elevation

- Topographic Map – Label the Lines

- Cut/Fill Calculations

  - Cut/Fill Ratio:   When soil is moved it settles.  When calculating the cuts and fills more soil will need to cut than filled.   A typical soil might require 10-30% extra cut volume.

Example: Volume Calculation

- Assume a station (100’ grid) needs to be cut .3’ the volume would be:

Or

- Note that it does not take much of a cut to require hauling a lot of soil.