# Electrical Troubleshooting Activities Stations & Key

| Station | Tester |
| --- | --- |
| 1 | Multi-meter |
| 2 | Multi-meter |
| 3 | Multi-meter |
| 4 | Multi-meter |
| 5 | Multi-meter |
| 6 | Multi-meter |
| 7 | Multi-meter |
| 8 | Multi-meter |
| 9 | Multi-meter |
| 10 | DR Tester |
| 11 | Induction |
| 12 | Clamp-on |
| 13 | Multi-meter |
| 14 | Multi-meter |

# Station #1 – Resistance Testing of Lamps

| Lamp | Resistance (ohms) | Good/Bad |
| --- | --- | --- |
| A | 0 | Bad |
| B | 16 | Good |

# Station #2 – Wiring Testing

| Cable | Fault (circle) | Which Wire(s) |
| --- | --- | --- |
| A | a. Shorted b. Broken c. No Fault | Black |
| B | a. Shorted b. Broken c. No Fault | White |

# Station #3 – Switch Testing

Toggle Switch: Is the switch working (Yes / No)

## 3 way Switch:

| Switch Position | Circle connected terminals |
| --- | --- |
| 1 (up) | A-B A-C B-C |
| 2 (down) | A-B A-C B-C |

C is common terminal

# Station #4 – DC Voltage Testing

| Battery | Voltage | Good/Bad |
| --- | --- | --- |
| A | Depends | Good |
| B | Depends | Bad |

# Station #5 – DC Voltage Testing Lead Acid Battery

Test depends on battery condition.

# Station #6 – Auto/Trailer Testing

Both filaments are good.

# Station #7 – Wiring Testing

| Cable | Fault (circle) | Which Wire(s) |
| --- | --- | --- |
| C | a. Shorted b. Broken c. No Fault |  |
| D | a. Shorted b. Broken c. No Fault | Black Broken, Black shorted to Ground |

# Station #8 AC Voltage Testing

| Outlet (DR) | Slot | Voltage Readings |
| --- | --- | --- |
| A Hot & Neutral Reversed | Neutral – Ground | 120 |
|  | Neutral – Hot | 120 |
|  | Hot - Ground | 0 |
| B Correct | Neutral – Ground | 0 |
|  | Neutral – Hot | 120 |
|  | Hot - Ground | 120 |
| C No grounding | Neutral – Ground | 0 |
|  | Neutral – Hot | 120 |
|  | Hot - Ground | 0 |
| D No Grounding | Neutral – Ground | 0 |
|  | Neutral – Hot | 120 |
|  | Hot - Ground | 0 |

- Is outlet #A wired correctly?  Yes/No

- Is outlet #B wired correctly?  Yes/No

- Is outlet #C wired correctly?  Yes/No

- Is outlet #D wired correctly?  Yes/No

# Station #9 – Power Supply (Transformer)

- Test the primary for resistance.  Unplug first!   The primary is the side with the plug.  Is the primary good? Yes /No.

- Plug in the transformer and test for AC voltage.   What is the voltage? ________0______

- Plug in the transformer and test for DC voltage.   What is the voltage? ________5______

# Station # 10 – Duplex Receptacle Tester

| Outlet | Fault |
| --- | --- |
| A | Open Gnd |
| B | Open Gnd |
| C | Hot/Neutral Reversed |
| D | Correct |

# Station # 11 – Power Present

Identify the two

| Breaker | Outlet |
| --- | --- |
| 15 A | A |
| 20 A | B |

# Station # 12 – Current draw.

Start with the load off.  What does each wire measure?

| Wire | Current |
| --- | --- |
| Black | 0 |
| White | 0 |
| Bare or Green | 0 |

Now clamp the probe over the black wire and turn the load on.   Observe the readings.  With the load on measure the other two wires.

| Wire | Current |
| --- | --- |
| Black | Depends on load |
| White | Depends on load |
| Bare or Green | 0 |

Can you explain the readings?  No current when not on.   Ground does not carry current.  White/Black should be the same.   If load is a motor then starting current is higher.

# Station # 13 – Fuses

Fuse A is OK,  Fuse B is blown.

# Station #14 – Resistor

Resistance 220 Ohms.