Shop Safety Checklist

Working in groups complete the shop safety checklist below.  Be prepared to discuss the following:

- What are the issues you found?

- How important are the items that you found?

- What remedies would recommend?

- How might you prioritize remediation?

Rank each issue as follows: 1=minor little risk of injury, 2=some risk of injury, but minor injury, 3=high risk or major injury

|  | Pass | Fail | N/A | Rank |
| --- | --- | --- | --- | --- |
| #### General Shop |  |  |  |  |
| Safety glasses are required. |  |  |  |  |
| Hearing protection is provided for use of tools exceeding 85 db. |  |  |  |  |
| Aisles are properly indicated, and aisles, passageways and corridors are free of obstructions. |  |  |  |  |
| Exit signs are visible and directional exit signs provided. Exit signs have letters 6" high, 3/4" strip and 5 candlepower illumination. |  |  |  |  |
| A fire alarm is provided. |  |  |  |  |
| Exit doors and access to exits are not locked from exit side. |  |  |  |  |
| Condition of floors maintained in a clean and dry condition and free from obstructions and debris. |  |  |  |  |
| Ventilation is proper and adequate for the lab. |  |  |  |  |
| Fire extinguishers of proper types, adequate1y located and maintained (tested yearly). Maximum height 5', if under 40#, 3 1/2' if over 40#. |  |  |  |  |
| - Class A fire - wood, paper, cloth and rubber-foam and dry chemical recommended |  |  |  |  |
| - Class B fire - flammable liquid, gas, and grease - dry chemical, CO2 and foam recommended |  |  |  |  |
| - Class C fire - energized electrical – dry chemical & CO2 recommended |  |  |  |  |
| - Class D fire - combustible metals specific for the chemical |  |  |  |  |
| Non-exit doors are clearly identified "not an exit" or otherwise labeled for use. |  |  |  |  |
| Every opening, floor or platform 4 ft. or more above ground level has a standard guard rail 42" high with an intermediate rail and a toe board of at least 4 in. above the floor. Rail must withstand 200# thrust. |  |  |  |  |
| All stairways having more than four (4) risers have a standard hand railing 30" – 34” high. |  |  |  |  |
| All gears, moving belts, chains, and shafts, etc., are protected by permanent guards. |  |  |  |  |
| All stationary power tools are anchored to the floor. |  |  |  |  |
| Illumination is adequate. Good shadow-free lighting is required -- 30 foot-candles (fc) general area, 50 fc on work areas, 100 fc for delicate repair work. |  |  |  |  |
| A trained person (or persons) is available to render first aid. |  |  |  |  |
| First aid supplies approved by the health department and readily available. |  |  |  |  |
| Fire blankets are readily available. |  |  |  |  |
| Battery charging area will provide facilities for flushing and neutralizing electrolyte. |  |  |  |  |
| Respirators/Dust Masks are provided in dusty areas. |  |  |  |  |
| Face shields are provided at every power tool. |  |  |  |  |
| Safety lanes are provided around power and non-powered stationary tools- |  |  |  |  |
| Non-skid surfaces provided at power tools on smooth floors. |  |  |  |  |
| Bump hats or hard hats are available for appropriate applications. |  |  |  |  |
| Toe guards are available for appropriate applications. |  |  |  |  |
| Approved first aid supplies are readily available. |  |  |  |  |
| Washing facilities are provided and properly maintained. |  |  |  |  |
| Restroom facilities are provided if not readily available. |  |  |  |  |
| A keyed master disconnect switch for all power tools. |  |  |  |  |
| Safety areas are marked around stationary tools. |  |  |  |  |
| Stationary machines are arranged so that individuals are protected from hazards of other machines. |  |  |  |  |
| Tools are kept sharp, clean, and in good working order. |  |  |  |  |
| Drill presses are equipped with clamps or vises. |  |  |  |  |
| Shears are shielded. |  |  |  |  |
| #### Materials Storage |  |  |  |  |
| Pressure safety cans are used for flammable or combustible liquids. |  |  |  |  |
| Combustible wastes are kept in covered metal containers. |  |  |  |  |
| A fire resistive cabinet is used to store flammable liquids. |  |  |  |  |
| Stored wood and metal are stacked safely and solidly so they will not fall. |  |  |  |  |
| #### Ladders |  |  |  |  |
| All ladders are in good repair and stored properly. Ladders not exposed to elements, heat and dampness, hung horizontally, |  |  |  |  |
| All portable ladders have safety feet. |  |  |  |  |
| All fixed ladders meet design specifications. Rung to back clearance - 7 in., max. rung spacing - 12 in., caged if more than 20 ft. to a max. of 30 ft. |  |  |  |  |
| #### Electrical |  |  |  |  |
| Electrical cabinets and power room is accessible only to authorized personnel. |  |  |  |  |
| Circuit breakers and disconnects are clearly labeled. |  |  |  |  |
| All electrical receptacles, junction boxes and switch boxes are properly covered. |  |  |  |  |
| All electrical outlets and fixtures are grounded. |  |  |  |  |
| All stationary power tools and equipment are grounded. |  |  |  |  |
| All extension cords are grounded. Cords of J wire grounding type. |  |  |  |  |
| Portable power tools are grounded or tools are double insulated. |  |  |  |  |
| All electrical disconnects are identified (circuit breakers, fuse boxes, etc.) in boxes. |  |  |  |  |
| The signal units for heating, ventilating and air-conditioning are operational. |  |  |  |  |
| Overcurrent devices are provided on all electrical loads. |  |  |  |  |
| #### Hoists |  |  |  |  |
| Safety hooks are used on all chains and lifting devices. |  |  |  |  |
| Rated load plainly marked on the side of lifting equipment. |  |  |  |  |
| Stands are provided to block raised equipment. |  |  |  |  |
| #### Compressed Air |  |  |  |  |
| Compressed air plumbing is steel or copper (L or M) not PVC. |  |  |  |  |
| Hoses are in good condition. |  |  |  |  |
| Air used for cleaning is regulated to not more than 30 psi and chip guarded. |  |  |  |  |
| Safety valve in the air line is in good working order. |  |  |  |  |
| There are pressure gauges on the air lines. |  |  |  |  |
| Air compressor is drained frequently. |  |  |  |  |
| Air compressor fans and flywheels within 7 ft. of the floor are guarded with a guard having holes not greater than 1/2 in. in width. Compressor is placarded as “automatic equipment”. |  |  |  |  |
| Woodworking Equipment |  |  |  |  |
| Saw blades and cutterheads are sharp and in good condition. |  |  |  |  |
| Push sticks or push blocks are available. |  |  |  |  |
| The table saw is equipped with a hood, guard, anti-kick back and spreader. |  |  |  |  |
| The band saw has an adjustable guard above the blade rolls and the blade wheels are covered. |  |  |  |  |
| The jointer has a working automatic guard covering all sections of the cutting head. |  |  |  |  |
| Power miter saw is in good working condition. Guards work smoothly. Saw is secured to a bench or stand. |  |  |  |  |
| The radial arm saw is equipped with hood guard, anti-kickback, rotation sign, automatic return, and table extension or stop. |  |  |  |  |
| All saws have roller units or stands to assist moving material to the saw and removal after cutting. |  |  |  |  |
| #### Grinders |  |  |  |  |
| The grinder has a safety guard at the point of operation. |  |  |  |  |
| The grinder has guards to cover the spindle ends, nuts and flanges. |  |  |  |  |
| The work rests are adjusted to within 1/8 inch of the grinding wheel. |  |  |  |  |
| The adjustable tongue guard or spark deflector on grinder is adjusted to within 1/4" of the grinder wheel. |  |  |  |  |
| Maximum periphery exposure for a stationary grinder is 65 degrees. (Exposed distance between tool rest and tongue guard or spark deflector.) |  |  |  |  |
| Grinding wheel of the right type and in good condition with no cracks, wheel diameter over half -of original size. |  |  |  |  |
| Cooling container nearby and filled. |  |  |  |  |
| Wheel dresser available. |  |  |  |  |
| All portable grinders and saws have proper shielding in position. Power cords in good condition. |  |  |  |  |
| The forge or small furnace is adequately vented. |  |  |  |  |
| #### Arc Welding |  |  |  |  |
| Adequate ventilation is provided for the dissipation of exhaust gases and welding fumes. |  |  |  |  |
| Arc welder cables are not worn or damaged. |  |  |  |  |
| Electrode holders are in proper condition to avoid exposure to electrical connections. |  |  |  |  |
| Electrode holders are hung up and so placed and fastened securely to the cable that they do not make electrical contact. |  |  |  |  |
| Arc welding helmets with tempered safety glass of the proper shade are in good condition. |  |  |  |  |
| - Shielded metal-arc welding (SMAW)1/16 - 5/32 electrodes No.10 |  |  |  |  |
| - TGAW 1/16 - 5/32 electrodes No. 11-12 |  |  |  |  |
| - Gas-shielded ferrous arc welding (GMAW) 1/16 -5/32 No. 12 |  |  |  |  |
| - Carbon arc. No. 14 |  |  |  |  |
| Fire resistant curtains or shields are used around arc welding areas or booths. |  |  |  |  |
| The floor in the welding area is kept dry and free of combustible aterials. |  |  |  |  |
| Protective gloves are provided and are in good condition. |  |  |  |  |
| Protective clothing such as jackets, sleeves, or capes are provided for out of position welding. |  |  |  |  |
| Shade 3-5 goggle or face shields are provided for plasma cutting. |  |  |  |  |
| #### Compressed Gas Storage |  |  |  |  |
| Compressed gas cylinders are chained or secured in place. |  |  |  |  |
| The protective cap is in place on all stored cylinders. |  |  |  |  |
| Combustible compressed gas and oxygen cylinders are separated by at least 20 ft. or a 5 ft. high wall of 1/2 hr. heat barrier. |  |  |  |  |
| #### Gas Welding |  |  |  |  |
| Portable gas welding equipment has the cylinder valves turned off when not in use. |  |  |  |  |
| Gas welding goggles with tempered safety glass of the proper shade are in good condition. |  |  |  |  |
| - Torch brazing No.3 or 4 |  |  |  |  |
| - Light cutting to 1 in. No.3 or 4 |  |  |  |  |
| - Gas welding (light) up to 1/8 in. No.4 or 5 |  |  |  |  |
| - Gas welding (med.) 1/8 - 1/2 in. No.5 |  |  |  |  |
| Oxyacetylene manifold welding systems have been installed according to the N.F.P.A. Shut off valves are working. |  |  |  |  |
| #### Paint |  |  |  |  |
| Paint spray booth has adequate ventilation. |  |  |  |  |
| Paint spray booth has proper lighting. |  |  |  |  |
| Explosion proof wiring within 20 feet of spray paint booth. |  |  |  |  |
| No open flames within 20 feet of opening of spray paint booth. |  |  |  |  |
| Spray paint area not littered with combustible materials. |  |  |  |  |
| #### Dress (does the class comply?) |  |  |  |  |
| Rings and other jewelry are removed when working in the shop. |  |  |  |  |
| All clothing worn is free from loose sleeves, flopping ties, loose coats. etc. |  |  |  |  |
| Long hair is tied back. |  |  |  |  |
| Shoes cover feet (closed toe). Leather is required for welding. |  |  |  |  |

#### Notes:

Adapted from Planning, Organizing, and Teaching Agricultural Mechanics.  Bear and Hoerner, Hobar Publications.  1986