Name ______________________

# EM-38 Sampling

Purpose:  To collect data using a sensor and map in GIS.  To understand how serial communications work and what issues are involved in this common communication method.

Equipment:

EM-38 instrument

Recon Unit with HGIS

Trimble Ag 124 GPS & Backpack.  Extra batteries

Farm Field Layer

Manuals for HGIS, EM-38, Trimble 124, Serial Primer

Set-Up

- Connect the EM-38 to a computer and use hyperTerminal to determine if the data is being sent.  What is the serial protocol being used?

Baud _____________

Bits ______________

Stop Bits: _________

Parity: ____________

- Connect the Recon unit to the Trimble GPS (A port) and establish a connection using HGIS.   What COM port and serial protocol is being used?

COM: _____________

Baud _____________

Bits ______________

Stop Bits: _________

Parity: ____________

- Connect the EM-38 to the Trimble GPS (B port) and program the GPS to pass the EM-38 data to the “A” port.

- Describe the cable configuration used below:

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

- Setup the Recon (HGIS) unit to collect the EM-38 data automatically every 2m.  Describe the setup steps below.

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

- Describe how you will collect data in the field (eg. What field, pattern, etc.)

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

__________________________________________________________________

Procedure (data collection):

- Soil should be at uniform moisture -- need to do after a rain.

- Assemble the EM-38, GPS, and Recon unit

- Load the Farm Field Map (for reference)

- Create a new layer using the EM-38 template

- Calibrate the EM-38 as described in the manual.

- Set the auto collect mode to 2m and 60 seconds.

- Taking turns pulling the EM-38 using the “auto collect mode to collect data.

- Save the layer in UTM.

- Download the layer.

Procedure (data analysis):

- Create a map using the collected point data.  Classify the points.  Use the Farm Field and soils map as a base layers.

- Save map as a jpg.

- Complete this lab sheet in Word and add your maps.

- Turn in Vista.

[Insert map here]