Name ______________________

# Variable Rate Technology

Variable Rate Technology (VRT) combines GPS and GIS with material application equipment to change rates of application based on a “prescription” map.  VRT can be used to apply materials more effectively, reduce materials use, and avoid material application in sensitive areas (buffers).

Purpose: To determine performance of a VRT system and understand the equipment and variables involved in VRT applications.

Equipment:

VRT Sprayer

Recon/GPS

Farm Works Software

Laptop (for data download)

Manuals for Raven & Farm Works (online)

Procedure:

- Construct a block diagram (see sample) with the direction of information flow and what information is exchanged for the following control system components:

  - VRT Controller/Software (note make/model)

  - GPS (note make/model)

  - Spray Controller (note make.model)

  - Flow Meter

  - Control Valve

  - Speed Sensor (note type)

- Determine the programmed boom width? _______________________ ft

- Mark the boundary of each test zone.

- Test the system at 4 MPH and 6 MPH (Ok to use tractor speedometer) using the vrt_test.shp application map. (see attached map).  Collect the as applied data.

- Record your observations.

On Your Own

- In ArcMap plot the prescription and the as applied maps (to be posted on Vista).  Classify the data for your analysis.

- Lab report is due in lab (do not turn in online).

Questions:

- What is the expected change in sprayer flow (GMP) between 4 and 6 MPH?

_____________________________________________________________

- Compare the as applied data at both speeds to the prescription and answer the following:

  - How well does the sprayer match the prescription?

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________

  - What pattern if any do you see in the as applied rates?

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________

  - Does speed make a difference

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________

  - Does the zone size make a difference?

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________

  - Does the magnitude of the rate change make a difference?

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________

Sample Block Diagram

Spray Controller

Flow Meter

Speed Sensor

Flow Control Valve

Speed

Flow

Valve Position

User Input

Rate, Booms, etc.

Useful Info:

GPM (per nozzle) = GPA X MPH X W ÷ 5940

GPA = Gallons / Acre

W=width in inches, nozzle spacing

GPM (Boom) = GPA X MPH X W ÷ 495

GPA = Gallons / Acre

W=width in feet

Notes:

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________

_____________________________________________________________