Name ______________________

Lab # 2 -- Tool Sharpening Template

Description:

The tool sharpening template demonstrates skill in project layout.  In addition, the template is a useful tool and will be used for the tool sharpening lab.

Materials:

22 ga. Galvanized sheet metal, approximately 4" x 7"

Tools:

Combination Square

Scribe

Snips

Metal punch

Sheet metal gauge.

Tool Sharpening Template

Name ________________________

Directions:

- Review the drawing and the grading criteria.

- Use the sheet metal gauge to verify the thickness of the material,

- Use the square to create a square corner on the upper right corner (4" side on top) of the sheet metal.  Trim to square if necessary.

- Make all measurements from the right or top edges. Total length is 5 1/2".

- Layout the project using the diagram.  Measure carefully.

- Cut out the project using snips.

- Flatten, if needed

- Use the scribe to mark the scale as shown on the template.

- Punch a hole in the bottom of the handle centered about 1/2" from the bottom.

- Using the scribe, write your name on the project and hand in with this CLEAN sheet.

Grading:

| Criteria (Tolerance 1/32") | Possible | Score |
| --- | --- | --- |
| Length 5 1/2" | 3 |  |
| Width 2 9/16" | 3 |  |
| Handle Width 1" | 3 |  |
| Center punch "V" | 3 |  |
| Cold Chisel "V" | 3 |  |
| Plane Iron "V" | 3 |  |
| Drill bit angle 59o | 3 |  |
| Cutting (minimum over cut, cuts on line) | 4 |  |
| Marking Scale | 2 |  |
| General appearance and workmanship | 3 |  |
| TOTAL | 30 |  |