# Station 1 - Switch

- Switch is a:

  - Single pole,  single throw switch

  - Single pole, double throw switch

  - Double pole, double throw switch

  - Double pole, single throw switch

# Station 2 – Time Clock

- What terminals are used for Normally Open operation?

  - N, Load 1, Line 1

  - N, Load 2, Line 2

  - N, Line 1, Load 2

  - N, Line 2, Load 1

# Station 3 – Photo Control

- What color wire is the Load?

  - Red

  - Black,

  - White

- What color wire is the Line?

  - Red

  - Black,

  - White

- What color wire is the Neutral?

  - Red

  - Black,

  - White

# Station 4 – Century Motor

- The motor horsepower is:

  - 1.0

  - 6.0

  - 12.0

  - 1.4

  - None of the above

- The motor rotation speed is:

  - 1700

  - 1725

  - 1750

  - 3450

- The motor shaft size is:

  - ½”

  - 5/8”

  - 9/16”

  - ¾”

- The minimum wire size to connect this motor at 115V is:

  - 16 gauge

  - 14 gauge

  - 12 gauge

  - 10 gauge

# Station 5 – Westinghouse Motor

- What is the current draw at 460 volts?

  - 1.0 A

  - 3.6 A

  - 1.8 A

  - 3 A

- What is the resistance of the winding connected to terminal A and B?  Use your multi-meter.  Choose the closest answer.

  - 0 ohms

  - 2.3 ohms

  - 4.9 ohms

  - 9.8 ohms

  - > 20 ohms

# Station 6 – Motor controller

- What is the maximum horsepower this unit can switch at 230 volts?

  - 1.0

  - 1.5

  - 2.0

  - 3.0

- What type of power does this unit switch?

  - 1 phase

  - 2 phase

  - 3 phase

  - 600 phase

# Station 7 – GE Motor

- What is the motor voltage?

  - 110

  - 115

  - 1725

  - 48

- Using your multi-meter test the winding.  Is it broken?

  - Yes

  - No

# Station 8 -- Pressure Switch

- What is the horsepower rating of the switch for a 1 phase 220V load?

  - ¼”

  - 1

  - 32

  - 110

- The motor (load) should be connected to which two terminals:

  - A and B

  - B and C

  - C and D

  - A and D

# Station 9 – Relay

- Use your multi-meter to test the coil (terminals D and E).  Is the coil broken?

  - Yes

  - No

# Station 10 – Photo Control

- What is the maximum load?

  - 120 W

  - 120 V

  - 2000 W

  - 2000 V

# Station 11 – Contactor

- How many poles is this switch?

  - 1

  - 2

  - 3

  - 4

  - 6