| Obtain permission from instructor before operating this machine |  | ## Band Saw |
| --- | --- | --- |

##### Safety suggestions

Saw irregular shapes

wood

plastic

Wear approved                                                                                                                                                                                                                                                                      eye protection.

- Eliminate loose clothing, confine long hair and remove jewellery from hands.

- Adjust the blade guard to within ¼” (6mm.) of the work.

- Remove cuttings only after the blade has stopped moving.

- Avoid backing out of the cut.

- Never place your fingers near the cut line.     Maintain a safe distance                                                   ( ____ “,            ____ mm., minimum) between your hands and the blade.                                                                               Use a push stick when necessary.

- Consult your instructor when cutting large or irregularly shaped pieces.

Do not leave the machine until after the blade has stopped moving.

Upper wheel guard

Blade support

Blade guard

Blade guides

Guide post

Table

Blade

Table insert

Mitre gauge groove

Table clamp

Band Saw

### Name    …………………..

### Class  …………………….

### Date ………………………

# Safety Quiz

| 1. | It is safe to tilt the table when cutting bevels. | T | F |
| --- | --- | --- | --- |
| 2. | The saw should be stopped by forcing a piece of wood against the blade. | T | F |
| 3. | The blade guard should be adjusted to within ¼” (6mm.) of the work. | T | F |
| 4. | The blade guides should be adjusted tight against the blade. | T | F |
| 5. | The hands should come no closer than 4” (10 cm.) from the blade. | T | F |
| 6. | All adjustments should be made with the power turned off. | T | F |
| 7. | Never place your fingers on the line you plan to cut. | T | F |

4.

1.

8.

9.

10.

5.

2.

3.

6.

7.