| Obtain permission from instructor before operating this machine |  | ## Grinder |
| --- | --- | --- |

Safety suggestions

shape ferrous metals

Eye protection must be worn at all times.

- Ensure that all guards are in place.

- The tool rest should be within 1/8” (3mm.) of the wheel.

- Do not grind materials on the side of the wheel.

- Small pieces objects must be held with “vise-grip” type pliers.

- Do not use the grinder if the wheel is damaged.

- The transparent safety shield should be clean.

Safety shield

Tool rest

Switch

Motor

Dust chute

Pedestal

Grinding wheel

Wheel guard

Spark deflector

Water pot

Stand to one side when you start the machine.

Grinder

### Name    …………………..

### Class  …………………….

### Date ………………………

# Safety Quiz

| 1. | The tool rest should be adjusted to within 1/8” (3mm.) of the wheel. | T | F |
| --- | --- | --- | --- |
| 2. | Eye protection is not necessary when grinding. | T | F |
| 3. | You may leave the machine as soon as the switch is in the “Off” position. | T | F |
| 4. | Bevel cutting tools should be ground to 25 deg. or 2x their thickness. | T | F |
| 5. | The safety shield should be clean. | T | F |
| 6. | Damaged wheels are safe to use. | T | F |
| 7. | Use “vise-grip” type pliers to hold small pieces. | T | F |

1.

2.

3.

4.

5.

6.

7.

8.

9.

10.