| Obtain permission from instructor before operating this machine |  | ## Radial Arm Saw |
| --- | --- | --- |

Safety suggestions

Cutting wood to length

Eye protection must be worn at all times.

- Ensure that all machine guards are in place and that the saw travels freely on the arm                  before turning on the saw.

- Adjacent to the blade, the stock must be flat on the                                                       table and its back edge supported by the fence.

- Apply pressure on to the fence with your left hand                                                 while controlling the saw with your right.

Arm

Handle

Kickback guard

Blade guards

Fence

Return the saw to the rear of the table on                                                                 completion of the cut.

.

Radial Arm Saw

### Name    …………………..

### Class  …………………….

### Date ………………………

# Safety Quiz

| 1. | Stock must rest securely on the table and along the fence. | T | F |
| --- | --- | --- | --- |
| 2. | The saw tends to self-feed toward the operator. Fence Feed Direction | T | F |
| 3. | Apply pressure toward the fence with your left hand when cross cutting. | T | F |
| 4. | Maintain a minimum distance of 4” (100 mm.) between your left hand and the blade path. | T | F |
| 5. | Return the saw to the rear of the table on completion of the cut | T | F |
| 6. | You may leave the machine as soon as the power is turned “Off”. | T | F |

5.

4.

3.

1.

2.